GMAT Portfolio — Scenario 08

Finite Burn + Fuel Budget: Hohmann Transfer 400 km → 600 km

Script Analysis and Explanation  ·  08_hohmann_dc.script

Prepared by Stephen O. Ogodo  ·  October 2026

# 1. Scenario Overview

Scenario 08 demonstrates finite burn manoeuvre planning for a two-burn Hohmann transfer between two circular low Earth orbits. The spacecraft begins in a 400 km circular parking orbit and is raised to a 600 km circular operational orbit using a 50 N bipropellant thruster. Three GMAT Differential Corrector (DC) targeting blocks are used sequentially to find the exact burn durations that achieve the target orbital elements.

The scenario illustrates the key difference between impulsive and finite burn manoeuvre planning: whereas an impulsive ΔV can be computed analytically from the vis-viva equation, a finite burn duration must be found numerically because the thrust arc straddles the apse, the spacecraft mass changes continuously, and gravity losses reduce the effective ΔV delivered.

# 2. Spacecraft Configuration

## 2.1 Orbital State

The spacecraft is initialised in a circular 400 km parking orbit with the following Keplerian elements:

| Parameter | Value |
| --- | --- |
| Epoch | 01 Jan 2024 00:00:00.000 UTC |
| Coordinate System | EarthMJ2000Eq (Earth-centred J2000 inertial) |
| State Type | Keplerian |
| SMA | 6778.137 km (Earth radius 6378.137 + 400 km altitude) |
| ECC | 0.0 (perfectly circular) |
| INC | 28.5° (mid-latitude launch — typical for Cape Canaveral) |
| RAAN | 0.0° |
| AOP | 0.0° |
| TA | 0.0° (burn begins at perigee) |

Setting TA = 0° places the spacecraft exactly at the perigee of what will become the transfer ellipse — this is the correct point to apply Burn 1 in a Hohmann transfer. Inclination of 28.5° is representative of a direct-ascent launch from Kennedy Space Center.

## 2.2 Mass Properties

| DryMass | 500.0 kg — structural, payload and subsystem mass |
| --- | --- |
| FuelMass | 100.0 kg — full propellant load at scenario start |
| Wet mass | 600.0 kg — total spacecraft mass at epoch |
| Cd | 2.2 — drag coefficient (standard sphere-cylinder approximation) |
| Cr | 1.8 — solar radiation pressure coefficient |
| DragArea | 15.0 m² — effective cross-sectional area for drag |
| SRPArea | 15.0 m² — effective area for solar radiation pressure |

The 100 kg propellant load represents a 16.7% propellant mass fraction — conservative and realistic for a LEO transfer vehicle. The pre-computation analysis shows that approximately 24.6 kg (24.6% of the tank) will be consumed across all three burns, leaving approximately 75.4 kg of residual propellant for subsequent missions or disposal manoeuvres.

## 2.3 Propulsion System

The thruster is modelled as a ChemicalThruster operating in the VNB (Velocity–Normal–Binormal) frame, directed purely along the +V axis (along-track, prograde direction). This is the correct direction for an orbit-raising Hohmann burn.

| Thruster type | 50 N bipropellant — MMH/NTO (monomethyl hydrazine / nitrogen tetroxide) |
| --- | --- |
| C1 (Thrust) | 50.0 N |
| K1 (Isp) | 290.0 s — conservative value for MMH/NTO at this thrust class |
| Exhaust velocity | 290.0 × 9.80665 = 2843.9 m/s |
| Mass flow rate | 50.0 / 2843.9 = 0.01758 kg/s (17.58 g/s) |
| Duty cycle | 1.0 — continuous firing (no pulse modulation) |
| DecrementMass | true — propellant consumption is tracked and deducted from TotalMass |
| Tank | FuelTank (ChemicalTank, pressure-regulated) |

Thruster Selection Rationale: The 50 N thrust level was chosen deliberately. A 22 N thruster — the most common small LEO biprop class (e.g. Aerojet R-6D) — would produce a Burn 1 arc of approximately 27% of the parking orbit period on a 600 kg spacecraft. This means the burn would not complete before the spacecraft passes apogee, breaking the Hohmann geometry entirely. At 50 N, the burn arc is approximately 12% of the orbital period (~660 seconds), keeping the finite-burn approximation valid while making the difference between impulsive and finite burn durations clearly visible in the GMAT output.

## 2.4 Fuel Tank

| FuelMass | 100.0 kg |
| --- | --- |
| Pressure | 2000.0 kPa (pressure-regulated biprop system) |
| Volume | 0.12 m³ (sized for 100 kg MMH at 866 kg/m³ density) |
| FuelDensity | 866.0 kg/m³ — monomethyl hydrazine (MMH) density |
| PressureModel | PressureRegulated — constant tank pressure throughout burn |
| AllowNegFuel | false — GMAT will abort if fuel runs out |

The PressureRegulated model is the correct choice here: in a regulated biprop system a pressure regulator maintains constant feed pressure to the thruster, so thrust and Isp remain essentially constant throughout the burn — consistent with the constant C1/K1 values in the thruster definition. A Blowdown model would model decreasing pressure and thrust as propellant is consumed, which is typical for monopropellant systems.

# 3. Force Model and Propagator

## 3.1 Force Model (DefaultProp_FM)

| Central body | Earth |
| --- | --- |
| Gravity model | JGM2 (Joint Gravity Model 2) |
| Gravity degree/order | 4×4 — captures J2 oblateness and first tesseral harmonics |
| Atmosphere | MSISE90 — active (drag is significant at 400–600 km) |
| SRP | Off — solar radiation pressure excluded for clarity |

Drag (MSISE90) is correctly enabled. At 400–600 km altitude the atmospheric density is sufficient for drag to accumulate meaningfully over multi-orbit timescales. Excluding drag at these altitudes would be a modelling error. The MSISE-90 model (Mass Spectrometer and Incoherent Scatter Radar Exosphere 1990) provides empirical density as a function of altitude, solar activity and geomagnetic index — suitable for this engineering-level analysis.

SRP is excluded because the scenario completes in approximately two to three orbital periods (roughly four to five hours). SRP accumulates over days to weeks and is negligible on this timescale.

Gravity degree/order 4×4 captures the dominant J2 term (Earth oblateness, which causes RAAN precession and argument-of-perigee drift) as well as the first tesseral and sectoral harmonics. For a LEO orbit-raising scenario this is more than sufficient — higher-order terms matter mainly for long-duration propagation.

## 3.2 Propagator

| Integrator | RungeKutta89 — 8th/9th order Runge-Kutta-Fehlberg |
| --- | --- |
| InitialStepSize | 60 s |
| MinStep | 0.001 s — prevents integrator stalling during burns |
| MaxStep | 2700 s — caps step size during long coasts |

RungeKutta89 is GMAT's highest-accuracy integrator for smooth force models. Its adaptive step-size control is critical during finite burns: as the spacecraft mass decreases and acceleration increases, the integrator automatically reduces its step size to maintain numerical accuracy without user intervention. The MaxStep of 2700 seconds (45 minutes) prevents the integrator from taking excessively large steps during the long coasts between burns.

# 4. Differential Corrector Targeting Strategy

## 4.1 Why Three Sequential DCs?

Three separate DC objects (DC1, DC2, DC3) are used in sequence rather than a single DC targeting multiple variables simultaneously. This is a deliberate design choice for numerical stability.

A simultaneous two-variable DC (e.g. varying both Burn1Dur and Burn2Dur to achieve both SMA targets together) requires inverting a 2×2 Jacobian matrix at each iteration. For a Hohmann transfer, this Jacobian is ill-conditioned because the two burns are physically coupled — changing Burn1Dur alters the transfer ellipse shape, which changes the conditions at apogee, which changes what Burn2Dur must achieve. The DC typically diverges.

Sequential 1×1 Jacobians are always well-conditioned: each DC has exactly one control variable and one constraint, so the Jacobian is a scalar and the Newton step is trivially computed. DC1 converges first; DC2 then runs from the converged state left by DC1; DC3 runs from the converged state left by DC2. Each DC runs to full convergence before the next begins.

## 4.2 DC1 — Perigee Kick to Transfer Ellipse

Targeting block: raises the apogee of the parking orbit to 600 km altitude.

Vary DC1(Burn1Dur = 693.1, {Perturbation = 0.5, Lower = 200, Upper = 1200, MaxStep = 30});

BeginFiniteBurn FinBurn1(SC);

Propagate DefaultProp(SC) {SC.ElapsedSecs = Burn1Dur};

EndFiniteBurn FinBurn1(SC);

Achieve DC1(SC.Earth.SMA = 6878.14, {Tolerance = 0.5});

| Control variable | Burn1Dur — duration of the perigee burn (seconds) |
| --- | --- |
| Initial guess | 693.1 s (Tsiolkovsky estimate of 660 s × 1.05 for gravity loss) |
| Perturbation | 0.5 s — finite-difference step for Jacobian estimation |
| Lower / Upper | 200 / 1200 s — physical bounds (prevents non-physical solutions) |
| MaxStep | 30 s — maximum change per DC iteration (prevents divergence) |
| Achieve target | SC.Earth.SMA = 6878.14 km — transfer ellipse SMA |
| Tolerance | 0.5 km — convergence criterion |

At convergence, the spacecraft is on a Hohmann transfer ellipse with perigee at 400 km and apogee at 600 km. The SMA of this ellipse is (6778.137 + 6978.137) / 2 = 6878.137 km, consistent with the DC1 Achieve target of 6878.14 km (rounded to two decimal places in the script).

Note that SC.Earth.SMA is the only reliable Achieve parameter for orbital targeting in GMAT. RadApo/RadPer are silently ignored in some GMAT versions; Apoapsis/Periapsis as a scalar Achieve parameter causes a Jacobian inversion error. SMA is always the correct choice.

## 4.3 Inter-burn Coast

Propagate DefaultProp(SC) {SC.ElapsedSecs = 2700.0};

Propagate DefaultProp(SC) {SC.Apoapsis};

After Burn 1, the spacecraft coasts to the apogee of the transfer ellipse where Burn 2 must be applied. The propagation uses two steps: a fixed 2700 second coast (45 minutes, which brings the spacecraft close to apogee without overshooting it) followed by propagation to the SC.Apoapsis stopping condition (which finds the exact apogee passage). The two-step approach is necessary because GMAT's stopping conditions can sometimes trigger prematurely at orbit start — propagating a minimum distance first avoids this.

The half-period of the transfer ellipse from the pre-computation is approximately 2838 seconds (47.3 minutes), so the 2700 second initial coast covers about 95% of the coast arc before handing over to the apoapsis stopping condition.

## 4.4 DC2 — Apogee Circularisation

Targeting block: raises the periapsis from 400 km to 600 km, completing the Hohmann transfer.

Vary DC2(Burn2Dur = 674.8, {Perturbation = 0.5, Lower = 200, Upper = 1200, MaxStep = 30});

BeginFiniteBurn FinBurn1(SC);

Propagate DefaultProp(SC) {SC.ElapsedSecs = Burn2Dur};

EndFiniteBurn FinBurn1(SC);

Achieve DC2(SC.Earth.SMA = 6978.14, {Tolerance = 0.5});

| Control variable | Burn2Dur — duration of the apogee circularisation burn |
| --- | --- |
| Initial guess | 674.8 s (Tsiolkovsky estimate of 642.7 s × 1.05) |
| Achieve target | SC.Earth.SMA = 6978.14 km (600 km circular orbit SMA) |
| Tolerance | 0.5 km |

The impulsive ΔV for Burn 2 is 55.142 m/s — slightly less than Burn 1 (55.545 m/s) because the spacecraft is lighter after Burn 1 and the velocity difference at apogee is slightly smaller than at perigee for this transfer. After DC2 converges, the spacecraft is in a near-circular 600 km orbit with a small residual eccentricity left by the finite burn arc.

## 4.5 DC3 — Partial ECC Trim

Targeting block: reduces the residual eccentricity left by the finite burn arcs.

Vary DC3(Burn3Dur = 96.4, {Perturbation = 1.0, Lower = 1, Upper = 300, MaxStep = 10});

BeginFiniteBurn FinBurn1(SC);

Propagate DefaultProp(SC) {SC.ElapsedSecs = Burn3Dur};

EndFiniteBurn FinBurn1(SC);

Achieve DC3(SC.Earth.ECC = 0.003, {Tolerance = 0.001});

| Control variable | Burn3Dur — duration of the ECC trim burn |
| --- | --- |
| Initial guess | 96.4 s (≈15% of Burn 2 propellant equivalent) |
| Achieve target | SC.Earth.ECC = 0.003 (partial trim, not full circularisation) |
| Tolerance | 0.001 |

The ECC target is 0.003, not 0.0. This is a deliberate engineering compromise. Achieving ECC = 0.0 exactly would require a very precisely timed and sized burn that would cost additional propellant and complexity. An eccentricity of 0.003 corresponds to a perigee-to-apogee altitude variation of approximately 2 × 0.003 × 6978 km ≈ 42 km — operationally insignificant for most LEO missions and well within typical mission tolerance bounds.

Note that this burn does not have an impulsive ΔV analogue. It is a purely numerical correction found by the DC — the reason the DC approach is essential rather than just a convenience.

# 5. Mission Sequence and Report Structure

## 5.1 Sequence Summary

| Step | Event | Purpose |
| --- | --- | --- |
| 0 | Initial report | Record baseline state: SMA, ECC, altitude, mass, fuel |
| 1 — DC1 | Burn 1 at perigee (~693 s) | Raise apogee to 600 km — enter transfer ellipse |
| 2 | Coast 2700 s + to Apoapsis | Propagate to apogee of transfer ellipse (~47 min) |
| 3 | Report at apogee | Record mid-transfer state |
| 4 — DC2 | Burn 2 at apogee (~675 s) | Circularise at 600 km — complete Hohmann transfer |
| 5 | Report post-Burn 2 | Record state after circularisation |
| 6 | Coast to Apoapsis | Propagate to next apogee for trim burn |
| 7 | Report at apogee | Record pre-trim state (ECC visible here) |
| 8 — DC3 | Burn 3 (~96 s) | Trim ECC from ~0.01 to 0.003 |
| 9 | Report post-trim | Record final orbital state and fuel remaining |
| 10 | Coast one full orbit (5801 s) | Confirm stable orbit — no spontaneous decay |
| 11 | Final report | Verify SMA, ECC, altitude and fuel mass at end |

## 5.2 Report File

The ReportFile (BurnReport) records six parameters at each Report call:

- SC.UTCGregorian — calendar time at each event

- SC.Earth.SMA — semi-major axis (km), shows orbit raise steps

- SC.Earth.ECC — eccentricity, shows residual after burns and trim

- SC.Earth.Altitude — instantaneous altitude (km)

- SC.TotalMass — total spacecraft mass (kg), confirms propellant consumed

- SC.FuelTank.FuelMass — propellant remaining (kg), the fuel budget output

The report output at `08_finite_burn_report.txt` is the primary validation artefact. The fuel budget is read directly from the difference between the initial and final FuelMass values.

# 6. Key Physics Concepts

## 6.1 Hohmann Transfer — Impulsive Baseline

A Hohmann transfer is the minimum-energy two-burn manoeuvre between two circular co-planar orbits. The impulsive ΔVs are derived from the vis-viva equation v = √(μ(2/r − 1/a)) applied at perigee and apogee of the transfer ellipse:

| Parking orbit SMA (r₁) | 6778.137 km (400 km altitude) |
| --- | --- |
| Target orbit SMA (r₂) | 6978.137 km (600 km altitude) |
| Transfer ellipse SMA | (r₁ + r₂) / 2 = 6878.137 km |
| v₁ (parking orbit) | 7668.6 m/s |
| v_tx (transfer perigee) | 7724.1 m/s |
| v_tx (transfer apogee) | 7502.7 m/s |
| v₂ (target orbit) | 7557.9 m/s |
| ΔV₁ (Burn 1, perigee) | 55.545 m/s |
| ΔV₂ (Burn 2, apogee) | 55.142 m/s |
| ΔV total | 110.687 m/s |

These values are mass-independent: they depend only on the gravitational parameter μ and the orbit radii. They serve as the anchor for the Tsiolkovsky burn sizing that generates the DC initial guesses.

## 6.2 Tsiolkovsky Rocket Equation — Burn Duration Estimates

The Tsiolkovsky equation gives the propellant mass consumed for a given ΔV:

m_final = m_initial × exp(−ΔV / ve)    where ve = Isp × g₀

Δm = m_initial − m_final

t_burn = Δm / ṁ    where ṁ = F / ve

| ve (exhaust velocity) | 290.0 × 9.80665 = 2843.9 m/s |
| --- | --- |
| ṁ (mass flow rate) | 50.0 / 2843.9 = 0.01758 kg/s |
| Burn 1 Δm | 11.605 kg consumed |
| Burn 1 duration (est.) | 660.1 s (Tsiolkovsky, impulsive approximation) |
| Burn 2 Δm | 11.299 kg consumed |
| Burn 2 duration (est.) | 642.7 s |
| DC initial guess (Burn 1) | 693.1 s = 660.1 × 1.05 (+5% for gravity/steering loss) |
| DC initial guess (Burn 2) | 674.8 s = 642.7 × 1.05 |

## 6.3 Why Finite Burn Durations Exceed the Tsiolkovsky Estimate

Three physical effects cause the actual GMAT burn duration to be longer than the impulsive Tsiolkovsky estimate:

- 1. Gravity loss: during a finite burn the spacecraft is also in free fall. Part of the thrust vector acts against gravity rather than purely accelerating the spacecraft along track. The effective ΔV delivered per unit of propellant consumed is therefore less than the ideal Isp × g₀ limit.

- 2. Steering loss (finite arc effect): an impulsive burn is instantaneous at the apse — the thrust direction is perfectly aligned with the velocity vector at that single instant. A finite burn straddles the apse: the spacecraft travels along a curved arc during the burn, and the VNB frame rotates continuously. The thrust vector is no longer perfectly tangential throughout the burn, introducing a small off-axis component that does not contribute to the desired orbit energy change.

- 3. Residual eccentricity: because neither Burn 1 nor Burn 2 is perfectly apse-centred, the resulting orbit is not perfectly circular. DC3 corrects the residual ECC at the cost of additional propellant, adding to the total fuel budget.

The 5% markup on the Tsiolkovsky estimate used for the DC initial guesses (660 s → 693 s for Burn 1) accounts primarily for gravity loss. The actual GMAT-converged value will typically be within 5–15% of the Tsiolkovsky estimate for burn arcs in the 10–15% orbital period range.

## 6.4 Burn Arc Constraint

For the Hohmann transfer geometry to remain valid, the finite burn arc must complete before the spacecraft reaches the opposite apse. The rule of thumb is that the burn duration should not exceed approximately 20% of the orbital period.

| Parking orbit period (T) | 5553.6 s (92.6 min) |
| --- | --- |
| 20% arc limit | 1110.7 s (~18.5 min) |
| Burn 1 duration (est.) | 660.1 s = 11.9% of T ✓ |
| Burn 2 duration (est.) | 642.7 s = 11.6% of T ✓ |
| 22 N thruster (rejected) | ~1500 s = 27.0% of T ✗ — arc wraps past apogee |

This constraint was the key driver in selecting 50 N over 22 N for this scenario. With a 22 N thruster on a 600 kg spacecraft, even reducing the transfer to 400 → 600 km (the smallest reasonable altitude step for this scenario) gives a 27% burn arc — the burn would not complete before the spacecraft passed apogee, making DC convergence physically impossible.

# 7. Interpreting the GMAT Output

## 7.1 Report File Columns

The report file `08_finite_burn_report.txt` will contain six columns at seven time points:

| Column | Parameter | What to look for |
| --- | --- | --- |
| 1 | UTCGregorian | Time stamps confirm the coast durations are correct |
| 2 | SC.Earth.SMA | Steps from 6778 → 6878 → 6978 km across the three burns |
| 3 | SC.Earth.ECC | Rises from 0 → ~0.01–0.03 after Burn 2, drops to 0.003 after Burn 3 |
| 4 | SC.Earth.Altitude | Confirms 400 km → ~600 km (varies with ECC until Burn 3) |
| 5 | SC.TotalMass | Drops by ~Δm at each burn; confirms propellant accounting |
| 6 | SC.FuelTank.FuelMass | The fuel budget: initial 100 kg → final ~75 kg (est.) |

## 7.2 XY Plots

- AltitudePlot (SC.Earth.Altitude vs ElapsedSecs): shows the characteristic step profile of a Hohmann transfer — flat parking orbit, rising transfer ellipse, flat final orbit. The finite burn smears the step transitions compared to an impulsive plot.

- FuelPlot (SC.FuelTank.FuelMass vs ElapsedSecs): shows three discrete drops corresponding to the three burns. The slope during each burn reflects the constant mass flow rate (17.58 g/s).

## 7.3 DC Convergence

With ShowProgress = true on all three DCs, GMAT will print iteration-by-iteration convergence data to the message window. A well-conditioned DC typically converges in 3–8 iterations from a good initial guess. If a DC requires more than 20 iterations or diverges, the most likely cause is an initial guess outside the physical solution basin — check that the Tsiolkovsky estimate is reasonable and that the Lower/Upper bounds encompass the true solution.

# 8. Summary

Scenario 08 presents a complete finite burn Hohmann transfer from 400 km to 600 km using three sequential Differential Corrector targeting blocks. The key design decisions and their physical justifications are:

- 50 N / 290 s biprop thruster: the minimum thrust level that keeps both burn arcs below 20% of the orbital period on a 600 kg spacecraft, ensuring the Hohmann geometry remains valid.

- Sequential DC blocks (1×1 Jacobian): always well-conditioned; avoids the Jacobian singularity that arises from simultaneous multi-variable targeting of a coupled two-burn sequence.

- SMA as the Achieve parameter: the only GMAT parameter that reliably converges for orbit-raising manoeuvres. RadApo/RadPer and Apoapsis/Periapsis have known stability issues.

- ECC trim to 0.003, not 0.0: a practical engineering compromise that corrects the residual finite-burn arc eccentricity without spending disproportionate propellant on a perfect circularisation.

- MSISE90 drag enabled: physically correct for 400–600 km LEO; omitting drag at these altitudes would be a modelling error even on the short timescale of this scenario.

The total impulsive ΔV budget is 110.687 m/s. The estimated propellant consumption is approximately 24.6 kg (24.6% of the 100 kg tank), leaving ~75.4 kg for subsequent operations. GMAT DC will refine these estimates to the actual finite-burn values once the script is executed.

— End of Analysis —