GMAT Astrodynamics Portfolio

Scenario 04a

Co-planar Inter-Satellite Link Range Analysis

2 Satellites  |  45° In-Track Separation  |  24-Hour Propagation

Author:    Stephen O. Ogodo

Tool:      General Mission Analysis Tool (GMAT) R2022a

Date:      16 October 2026

# 1. Executive Summary

This report presents the results of a 24-hour GMAT simulation of two co-planar LEO satellites (LEOSat1 and LEOSat2) in identical circular orbits at 400 km altitude, separated by a 45-degree in-track angle at epoch. The primary output is a time series of the inter-satellite link (ISL) range computed every 60 seconds over the full simulation window.

The simulation confirms the expected starting chord geometry (5,187 km) and reveals a monotonic range decrease of 853 km over 24 hours — a slow convergence driven by the differential orbital dynamics of the two satellites. This baseline establishes the ISL range characteristics of the simplest possible two-satellite co-planar formation, against which the more complex six-satellite (Scenario 04b) and cross-planar (Scenario 05) configurations are compared.

Key finding: The ISL range decreases steadily from 5,187 km to 4,334 km over 24 hours at a rate of ~53 km per orbit. No oscillation occurs. The two satellites slowly converge — a conjunction trend that would require a station-keeping burn to correct over a longer timeframe.

# 2. Scenario Configuration

## 2.1 Orbital Parameters

Both satellites occupy the same circular orbit at ISS altitude. The only difference between them is the true anomaly at epoch, which sets the in-track angular separation.

| Parameter | LEOSat1 | LEOSat2 | Notes |
| --- | --- | --- | --- |
| Semi-major axis (SMA) | 6,778.14 km | 6,778.14 km | ~400 km altitude |
| Eccentricity (ECC) | 0.0001 | 0.0001 | Near-circular |
| Inclination (INC) | 51.6° | 51.6° | ISS inclination |
| RAAN | 0° | 0° | Same ascending node → co-planar |
| Arg. of Perigee (AOP) | 0° | 0° | Undefined for circular orbit |
| True Anomaly (TA) | 0° | 45° | 45° in-track separation |
| Epoch | 01 Jan 2024 00:00:00.000 UTC | 01 Jan 2024 00:00:00.000 UTC | Same epoch |

## 2.2 Propagator

GMAT DefaultProp with no explicit force model settings. GMAT R2022a applies point-mass Earth gravity, the Runge-Kutta 8/9 integrator, and a maximum step size of 2,700 seconds. Both satellites are propagated simultaneously in lock-step at 60-second intervals. No atmospheric drag or solar radiation pressure is modelled.

## 2.3 ISL Range Computation

GMAT has no built-in inter-satellite range variable. The range is computed directly in the mission sequence at each 60-second step using the Euclidean distance formula in the EarthMJ2000Eq inertial Cartesian frame:

ISL_Range = √[ (X₁−X₂)² + (Y₁−Y₂)² + (Z₁−Z₂)² ]

This gives the true 3-D scalar distance between the two spacecraft at each epoch. The value is written to the report file via the ReportFile subscriber at every propagation step.

## 2.4 Simulation Output

| Parameter | Value |
| --- | --- |
| Simulation duration | 24 hours (86,400 seconds) |
| Report cadence | 60 seconds |
| Total report rows | 1,441 (1 per minute + final state) |
| Output file | ISLReport.txt |
| Columns logged | Epoch (UTC), LEOSat1 X/Y/Z, LEOSat2 X/Y/Z, ISL_Range |

# 3. Results

## 3.1 ISL Range Statistics

| Metric | Value | Time / Notes |
| --- | --- | --- |
| Starting ISL range | 5,187.286 km | 01 Jan 2024 00:02:00 UTC |
| Ending ISL range | 4,334.487 km | 02 Jan 2024 00:02:00 UTC |
| Maximum range | 5,187.286 km | At epoch (t = 0) |
| Minimum range | 4,334.487 km | End of 24-hour window |
| Mean range | 4,764.512 km | Over 1,441 data points |
| Total drift | 852.799 km decrease | Monotonic over 24 h |
| Drift rate per orbit | ~53 km / 92.55 min orbit | ~0.57 km / min |

## 3.2 Orbit-by-Orbit Range Evolution

The table below shows the ISL range at the end of each successive orbit (approximately every 92.55 minutes), confirming the steady, monotonic decrease with no oscillation:

| Orbit # | Time (UTC) | ISL Range (km) | Change from Previous (km) |
| --- | --- | --- | --- |
| 1 (epoch) | 01 Jan 2024 00:02:00 | 5,187.286 | — |
| 2 | 01 Jan 2024 01:33:00 | 5,134.369 | −52.917 |
| 3 | 01 Jan 2024 03:05:00 | 5,081.277 | −53.092 |
| 4 | 01 Jan 2024 04:37:00 | 5,028.161 | −53.116 |
| 5 | 01 Jan 2024 06:09:00 | 4,975.059 | −53.102 |
| 6 | 01 Jan 2024 07:41:00 | 4,921.892 | −53.167 |
| ~15 (end) | 02 Jan 2024 00:02:00 | 4,334.487 | −852.799 total |

## 3.3 Starting Range Verification

The starting ISL range can be verified analytically using the chord-length formula for two points separated by an angular arc on a circle of radius r:

d = 2 × r × sin(θ / 2)  =  2 × 6,778.14 × sin(22.5°)  =  2 × 6,778.14 × 0.3827  =  5,187 km  ✓

The simulation result of 5,187.286 km matches the analytical chord formula to within 1 km — the small residual reflects the non-zero eccentricity (0.0001) and the 2-minute lag before the first ISL_Range computation in the mission sequence.

# 4. Analysis

## 4.1 Why the Range Decreases Monotonically

The monotonic range decrease is a fundamental consequence of having only two satellites in the same orbital plane with no counterbalancing third body. Under point-mass Earth gravity, the two satellites should maintain a perfectly constant angular separation indefinitely. The slow convergence observed in this simulation is the result of the numerical integration accumulating a small but non-zero differential phase error over 1,440 propagation steps.

In a real mission context with explicit J2 perturbations, the effect would be physically driven: J2 causes a J2-corrected mean motion that is slightly different from the Keplerian mean motion, and at 51.6° inclination the J2 effect on both satellites is nearly identical (same SMA, same INC). Any residual differential drift between LEOSat1 and LEOSat2 would arise from tiny initial-state differences or higher-order gravity terms.

The key point is that with only two satellites, there is no third body to "catch" one satellite as it drifts — the gap closes in one direction only, producing the monotonic trend visible in the data.

## 4.2 Contrast with Scenario 04b (6 Satellites)

In Scenario 04b with six co-planar satellites separated by 60° each, the ISL ranges oscillate between 5,148 km and 7,882 km, with some pairs exceeding the 7,500 km ISL threshold. This oscillation arises because the six satellites form a closed constellation: as one pair converges, the complementary pair diverges, creating a periodic "breathing" pattern in the range data.

With only two satellites, no such complementary pair exists. The two-satellite result is therefore not representative of a real constellation — it is a limiting case that illustrates the geometric baseline and the fundamental instability of an isolated two-body formation.

| Metric | Scenario 04a (2 Sats, 45°) | Scenario 04b (6 Sats, 60°) |
| --- | --- | --- |
| Starting range | 5,187 km | ~6,770 km (60° chord) |
| Range behaviour | Monotonic decrease | Oscillating |
| Max range | 5,187 km | 7,882 km |
| Min range | 4,334 km | 5,148 km |
| ISL threshold risk | None (always < 5,200 km) | Yes (pairs exceed 7,500 km) |
| Primary risk | Conjunction (convergence) | Link dropout (divergence) |

## 4.3 Operational Significance

From an operations perspective, the two-satellite scenario presents a different risk profile from a multi-satellite constellation:

- Link availability: The ISL range stays well below any practical RF threshold throughout the 24-hour window. Link dropout is not a concern for this configuration.

- Conjunction risk: The slow convergence (853 km / 24 h = 35.5 km / hour) would bring the satellites closer over successive days. Without correction, the in-track separation shrinks from 45° toward 0° on a timescale of weeks to months (extrapolating linearly: 45° / 53 km per orbit × chord factor ≈ several hundred orbits ≈ 2–3 weeks).

- Station-keeping requirement: A prograde or retrograde burn on one satellite (adjusting its mean motion slightly) would be required to restore the 45° spacing — the same class of manoeuvre used for constellation slot maintenance in operational LEO systems.

- Applicability to NigComSat context: NigComSat-1R operates in GEO, where two-body co-planar ISL geometry does not directly apply. However, the principles demonstrated here — chord geometry, differential drift, and the need for formation-keeping manoeuvres — are directly transferable to the LEO extension scenarios in this portfolio.

# 5. Conclusions

Scenario 04a establishes the inter-satellite link range baseline for the simplest possible co-planar LEO formation: two satellites, same orbit, 45° angular separation.

The simulation confirms:

- The starting ISL range of 5,187 km matches the analytical chord formula (2r sin θ/2) to within 1 km, validating the GMAT setup and the in-script range computation.

- The range decreases monotonically by 853 km over 24 hours at a steady rate of ~53 km per orbit with no oscillation — a fundamental difference from the six-satellite case.

- The two-satellite formation is inherently unstable against conjunction: without periodic station-keeping, the in-track gap would close on a timescale of weeks.

- The ISL range stays well below any practical link threshold throughout the simulation, confirming adequate link availability for the 24-hour window.

These results feed directly into the comparative analysis in Scenario 04b and Scenario 05, where the addition of more satellites and a cross-plane geometry introduce oscillatory behaviour and the risk of ISL threshold violations.

# 6. GMAT Script Reference

| Item | Value |
| --- | --- |
| Script file | 04a_2_co-planar_satellites.script |
| Commented script | 04a_2_co-planar_satellites_commented.script |
| Output report | ISLReport.txt |
| GMAT version | R2022a |
| Simulation duration | 24 hours |
| Step size | 60 seconds |
| Data rows | 1,441 |
| Propagator | DefaultProp (RK89, point-mass Earth gravity) |
| Coordinate frame | EarthMJ2000Eq (J2000 inertial) |