GMAT ASTRODYNAMICS PORTFOLIO

Scenario 09b: GEO East-West Station-Keeping

Full Two-Burn Operational Cycle

NigComSat-1R @ 42.5 degE  |  GMAT R2022a  |  33.5-Day Simulation

Author: Stephen O. Ogodo  |  Tool: GMAT R2022a  |  Date: October 2026

1. Executive Summary

Scenario 09b extends the single-burn EWSK demonstration of Scenario 09a into a complete two-burn operational cycle — the minimum unit of repeating station-keeping practice for a GEO satellite. Two prograde burns are executed for NigComSat-1R at 42.5 degE, each triggered by the 24-hr mean longitude crossing the 42.53 degE east trigger. The full 33.5-day simulation encompasses: an initial 1.5-day coast to the first trigger, Burn 1 (11 s), a 15-day westward arc, Burn 2 (7 s, SMA-matched), and a 17-day second westward arc ending at the east trigger where Burn 3 would fire in a continuing operational cycle.

The critical engineering insight demonstrated in this scenario is the SMA-matching principle: Burn 2 is not a copy of Burn 1. During the 15-day coast, the satellite's semi-major axis (SMA) decays from 42167.2 km back toward synchronous, reaching 42165.2 km before Burn 2. A naive repeat of the 11 s Burn 1 duration would over-raise the SMA to 42168.6 km, causing a westward runaway violation. The correct approach — proven here — targets the same post-burn SMA (42167.2 km) for both burns, computing Burn 2 duration from the actual pre-burn SMA: (42167.2 - 42165.2) / 0.309 km/s = 6.47 s, rounded up to 7 s.

| Metric | Value | Compliance |
| --- | --- | --- |
| Mean longitude range | 42.472 degE to 42.534 degE | PASS (deadband: 42.45-42.55 degE) |
| Instantaneous lon range | 42.420 degE to 42.582 degE | PASS (diurnal oscillation: evaluated on mean) |
| EWSK cycle period | 15.0 days | PASS (consistent with NigComSat-1R ops) |
| Fuel consumed (2 burns) | 0.167 kg of 800 kg | PASS (99.98% fuel remaining) |
| Scenario duration | 33.5 days (2 full cycles observed) | COMPLETE |
| Deadband violations | 0 | PASS |

2. Mission Background and Operational Context

2.1 NigComSat-1R and the 42.5 degE Slot

NigComSat-1R is a Nigerian communication satellite operated by Nigerian Communication Satellite Ltd (NigComSat), located at the 42.5 degE geostationary orbital slot — an ITU-filed position that cannot be vacated. The satellite operates in C, Ku, Ka, L, and X bands, providing communications across Africa and the Middle East.

East-West Station-Keeping (EWSK) is required to maintain the satellite within its ITU-mandated deadband of +/-0.05 degrees (42.45 degE to 42.55 degE). Without EWSK, tesseral harmonic perturbations from Earth's non-spherical gravitational field would drive the satellite away from the slot, violating ITU filing requirements and causing interference with adjacent satellites.

2.2 Relationship to Scenario 09a

Scenario 09a (v17) demonstrated a single pre-emptive prograde burn that:

- Fires at the 42.53 degE east operational trigger (before the 42.55 degE hard limit)

- Raises SMA above synchronous (42167.2 km) to reverse drift from eastward to westward

- Produces a 15-day compliant westward arc — bottom at 42.487 degE mean longitude

Scenario 09b builds directly on 09a, using identical spacecraft, force model, propagator, thruster, and initial burn parameters. The new contribution is the second burn — and the scientific derivation of its duration and timing.

2.3 GEO Synchronous SMA and Drift Physics

A key reference quantity for all EWSK analysis is the synchronous SMA — the semi-major axis at which orbital period exactly equals one sidereal day (86164.1 s):

SMA_sync = (mu * T_sidereal^2 / 4*pi^2)^(1/3)

= (398600.4415 * 86164.1^2 / 4*pi^2)^(1/3)

= 42164.1696 km

The drift direction depends entirely on whether the satellite's SMA is above or below synchronous:

- SMA < sync (42164.17 km): period < sidereal day -> satellite orbits faster than Earth rotates -> eastward drift relative to Earth

- SMA > sync (42164.17 km): period > sidereal day -> satellite orbits slower than Earth rotates -> westward drift relative to Earth

- SMA = sync: satellite is stationary in longitude (geostationary ideal)

At 42.5 degE — near an unstable tesseral equilibrium — the longitude is between the two stable equilibrium points (75 degE and 255 degE). Tesseral harmonics provide a gentle eastward restoring force when the satellite is at or near synchronous SMA, driving slow eastward drift that builds up over days.

3. Simulation Configuration

3.1 Spacecraft Parameters

| Parameter | Value | Notes |
| --- | --- | --- |
| Satellite name | NigComSat-1R (SC) | ITU-filed GEO asset at 42.5 degE |
| Epoch | 01 Jan 2024 00:00:00.000 UTC | GMAT UTCGregorian format |
| SMA at epoch | 42164.1696 km | Exactly synchronous — TA places sat at 42.5 degE |
| ECC | 0.0001 | Near-circular GEO orbit |
| INC | 0.05 deg | Residual inclination (realistic operational value) |
| RAAN | 0.0 deg | Reference |
| AOP | 0.0 deg | Reference |
| TA at epoch | 142.3452 deg | Pre-computed to place satellite at 42.5 degE at epoch |
| Dry mass | 1000.0 kg | Representative GEO satellite |
| Fuel mass (epoch) | 800.0 kg | Full tank at scenario start |
| Thruster thrust | 20.0 N (C1=20.0) | Chemical bipropellant (Spacebus class) |
| Thruster Isp (K1) | 220.0 s | Representative bipropellant Isp |
| SRP area | 50.0 m^2 | Large solar array |
| Drag area | 20.0 m^2 | GEO: negligible drag, included for completeness |
| Cr (reflectivity) | 1.8 | Standard reflectivity coefficient |

3.2 Force Model

| Force | Setting | Significance for EWSK |
| --- | --- | --- |
| Central body gravity | Earth (JGM2.cof) | Dominant force |
| Gravity degree/order | 4 x 4 tesseral | Captures C22/S22 terms that drive east-west drift at 42.5 degE |
| Solar radiation pressure | On (Cr=1.8, area=50 m^2) | Measurable effect at GEO; contributes ~0.01 deg/day eccentricity growth |
| Atmospheric drag | None | Negligible at GEO altitude (35786 km) |
| Relativistic correction | Off | Sub-mm/s effect; not required for EWSK |
| Third-body gravity | Not modelled | Sun/Moon third-body relevant for NSSK (inclination) only, not EWSK |

3.3 Propagator

| Parameter | Value |
| --- | --- |
| Integrator | RungeKutta89 (8th-9th order adaptive) |
| Initial step size | 60 s |
| Accuracy | 1e-13 (tight tolerance for GEO precision) |
| Min step | 0.001 s |
| Max step | 86400 s (1 day) |
| Max step attempts | 50 |

3.4 Output Configuration

The GMAT ReportFile records six quantities at every integration step:

- SC.UTCGregorian — date/time stamp (parsed to elapsed days in post-processing)

- SC.Earth.Longitude — instantaneous sub-satellite longitude (degrees East)

- SC.ECC — orbital eccentricity

- SC.SMA — semi-major axis (km) — the primary manoeuvre planning quantity

- SC.TotalMass — total spacecraft mass (dry + fuel, kg)

- SC.FuelTank.FuelMass — propellant remaining (kg) — burn detection signal

Output file: C:\Users\Admin\GMAT_Tutorials\GMAT_portfolio\09_scenario_EWSK\scenairo_09b\09b_geo_ewsk_report.txt — 3,661 data rows spanning day 0.0 to day 33.5.

4. Mission Sequence

The mission sequence implements five phases in GMAT:

| Phase | GMAT Command | Duration | Purpose |
| --- | --- | --- | --- |
| Phase 0: Initial coast | Propagate {SC.ElapsedDays = 1.5} | 1.5 days | Coast from 42.5 degE to east trigger at 42.53 degE |
| Burn 1 | BeginFiniteBurn FinBurn1 / Propagate {SC.ElapsedSecs = 11} | 11 s | Raise SMA from 42163.8 to 42167.2 km |
| Phase 1: Westward arc | Propagate {SC.ElapsedDays = 15.0} | 15.0 days | Westward coast, arc bottom ~42.487 degE mean lon |
| Burn 2 | BeginFiniteBurn FinBurn2 / Propagate {SC.ElapsedSecs = 7} | 7 s | Raise SMA from 42165.2 to 42167.3 km (SMA-matched) |
| Phase 2: Second arc | Propagate {SC.ElapsedDays = 17.0} | 17.0 days | Second westward coast, ends at east trigger (day 33.5) |

Stopping condition design: All propagation uses ElapsedDays/ElapsedSecs-based stopping only. Longitude-based stop conditions are explicitly avoided because the instantaneous longitude oscillates +/-0.035 deg with ~24-hr period (diurnal oscillation from residual eccentricity and inclination), which would cause premature or erratic trigger firing. The coast durations (1.5 days, 15.0 days, 17.0 days) are derived scientifically from post-processing the report data of prior runs — not chosen arbitrarily.

5. Burn Epoch Derivation

5.1 Burn 1 Epoch — Day 1.5

At epoch the satellite is placed at 42.5 degE (synchronous SMA=42164.17 km). The tesseral harmonic at 42.5 degE drives a gentle eastward drift — the satellite is near the unstable equilibrium and slowly accelerates eastward. From prior Scenario 09a analysis, the 24-hr mean longitude reaches the 42.53 degE east operational trigger at day 1.5. This is the correct burn epoch: fire before the 42.55 degE ITU hard limit is ever breached (pre-emptive station-keeping, not reactive).

5.2 Burn 2 Epoch — Day 16.5 (15.0 days after Burn 1)

After Burn 1, the satellite enters a westward coast arc. The 24-hr mean longitude profile from the 09b report data:

| Elapsed Day | 24-hr Mean Longitude (degE) | Phase |
| --- | --- | --- |
| 1.5 (Burn 1) | 42.5338 | East trigger — burn fires |
| 3.0 | 42.5189 | Westward, decelerating |
| 6.0 | 42.4954 | Approaching arc bottom |
| 9.0 | 42.4873 | ARC BOTTOM — westernmost point |
| 12.0 | 42.4947 | Recovering eastward |
| 13.0 | 42.5006 | Crosses slot centre 42.5 degE |
| 15.0 | 42.5175 | Approaching east trigger |
| 16.0 | 42.5285 | Closing on 42.53 degE |
| 16.5 (Burn 2) | 42.5328 | EAST TRIGGER CROSSED — Burn 2 fires |
| 17.5 (if no burn) | 42.5487* | *Would approach 42.55 degE hard limit |

Burn 2 epoch principle: Burn fires at the first rising crossing of the 42.53 degE east trigger by the 24-hr rolling mean longitude on the eastward recovery arc. In this scenario that crossing occurs at day 16.5 — exactly 15.0 days after Burn 1. This is not a fixed timer but a data-driven determination from the prior run's mean longitude profile.

EWSK cycle length: The 15.0-day burn-to-burn interval is consistent with the operational NigComSat-1R history. It is determined by the drift dynamics: how far west the arc goes, how fast the satellite recovers eastward, and where the east trigger is set.

6. Burn Duration — Generalised Framework

This is the central engineering contribution of Scenario 09b. The derivation establishes a generalised formula for any EWSK burn duration based on the satellite's actual SMA at the moment of firing.

6.1 Physical Constants from Burn 1 (Reference Burn)

Burn 1 (11 s prograde) gives the fundamental constant of the EWSK system:

dSMA/dt_burn = delta_SMA / burn_duration

= (42167.2000 - 42163.8000) km / 11 s

= 3.4000 km / 11 s

= 0.30909 km/s  [SMA raise rate per second of prograde burn]

This constant is a property of the thruster (thrust = 20 N, Isp = 220 s) and the spacecraft mass (~1000 kg), mediated by the Tsiolkovsky rocket equation. It is approximately invariant across burns as long as spacecraft mass does not change significantly.

6.2 SMA Target — Post-Burn Operational Value

After Burn 1, GMAT reports SMA = 42167.2000 km — which is 3.03 km above synchronous (42164.17 km). This post-burn SMA is not arbitrary: it is the value that produces a compliant ~15-day westward arc ending near the east trigger (42.53 degE mean longitude). It is the operational SMA target that all subsequent burns must restore:

SMA_target = 42167.2000 km  (= 42164.1696 + 3.0304 km above sync)

This is the proven value from the compliant Scenario 09b run.

It is the goal of every EWSK burn regardless of pre-burn SMA state.

6.3 Coast SMA Decay Rate

After Burn 1 raises SMA to 42167.2 km, the satellite is above synchronous — its orbital period exceeds one sidereal day, causing westward drift. As the satellite drifts westward, tesseral harmonics pull the SMA back toward synchronous (decaying, not growing). The GMAT report data confirms:

| Day | SMA (km) | SMA above sync (km) | Decay from post-B1 peak (km) |
| --- | --- | --- | --- |
| 1.5 (post-Burn 1) | 42167.2000 | 3.0304 | 0.0000 |
| 2.0 | 42167.3000 | 3.1304 | +0.1 (numerical, initial transient) |
| 3.0 | 42167.1000 | 2.9304 | -0.1 |
| 6.0 | 42166.7000 | 2.5304 | -0.5 |
| 9.0 | 42166.3000 | 2.1304 | -0.9 |
| 12.0 | 42165.9000 | 1.7304 | -1.3 |
| 15.0 | 42165.5000 | 1.3304 | -1.7 |
| 16.0 | 42165.4000 | 1.2304 | -1.8 |
| 16.5 (pre-Burn 2) | 42165.2000 | 1.0304 | -2.0 |

The SMA decays from 42167.2 km to 42165.2 km over 15.0 days — a total loss of 2.0 km. The decay rate is:

dSMA/dt_coast = (42165.2000 - 42167.2000) / 15.0 days

= -2.0000 / 15.0

= -0.13333 km/day

Note: SMA DECREASES during coast (decays back toward synchronous).

The satellite is still ABOVE synchronous (westward drift continuing),

but the SMA margin is eroding toward zero over ~22 more days if uncorrected.

6.4 The Generalised Burn Duration Formula

Combining the burn rate constant and the SMA target, the burn duration for any EWSK burn N is:

T_burn(N) [s]  =  (SMA_target - SMA_pre(N)) / (dSMA/dt_burn)

Where:

- SMA_target = 42167.2000 km (the proven operational post-burn SMA)

- SMA_pre(N) = actual SMA measured from the GMAT report at the burn N epoch

- dSMA/dt_burn = 0.30909 km/s (from Burn 1: 3.4 km / 11 s)

- Round UP to nearest integer second: overshoot SMA_target is safer than undershoot

Predictive formula for SMA_pre: If the coast duration T_coast is known, SMA_pre can be predicted without running GMAT first:

SMA_pre(N) = SMA_target + (dSMA/dt_coast) x T_coast(N)

= 42167.2000 + (-0.13333) x T_coast(N)

= 42167.2000 - 0.13333 x T_coast(N)  [km]

For T_coast = 15.0 days:

SMA_pre(2) = 42167.2000 - 0.13333 x 15.0 = 42167.2000 - 2.0 = 42165.2000 km

6.5 Formula Verification Against GMAT Data

| Burn | SMA_pre (km) | SMA_target (km) | Delta SMA (km) | Formula Duration (s) | Rounded Duration | Actual GMAT |
| --- | --- | --- | --- | --- | --- | --- |
| Burn 1 | 42163.8000 | 42167.2000 | 3.4000 | 3.4000 / 0.30909 = 11.00 s | 11 s (exact) | 11 s |
| Burn 2 | 42165.2000 | 42167.2000 | 2.0000 | 2.0000 / 0.30909 = 6.47 s | 7 s (rounded UP) | 7 s |

Burn 1 verifies the formula exactly (11.00 s — no rounding needed). Burn 2 demonstrates the rounding-up principle: the formula gives 6.47 s, which is rounded to 7 s. The extra 0.53 s produces a slight SMA overshoot (42167.3 km vs. target 42167.2 km — an overshoot of 0.1 km), which is operationally safe: it extends the westward arc slightly and delays the next east trigger crossing, but keeps the satellite well inside the deadband.

6.6 Why Burn 2 is NOT 11 Seconds

This is the key insight that differentiates a scientifically designed burn sequence from a naive copy-paste. If Burn 2 used the same 11 s duration as Burn 1:

Hypothetical Burn 2 at 11s:

SMA_post = 42165.2000 + (0.30909 x 11) = 42165.2000 + 3.4 = 42168.6000 km

SMA above sync = 42168.6000 - 42164.1696 = 4.43 km  (vs. 3.03 km target)

Extra above-sync margin = 1.40 km -> longer/deeper westward arc

-> Mean longitude would breach 42.45 degE WEST LIMIT -> VIOLATION

This was confirmed by a test run of the 11 s Burn 2 version — it produced a westward runaway violation beyond the 42.45 degE hard limit. The SMA-matching approach is therefore operationally critical, not optional.

7. Phase-by-Phase Results

7.1 Phase 0 — Initial Coast (Day 0 to Day 1.5)

| Quantity | Value |
| --- | --- |
| Epoch longitude | 42.5000 degE |
| Epoch SMA | 42164.2000 km (= synchronous) |
| Epoch fuel | 800.0000 kg |
| Duration | 1.5 days |
| Drift direction | Eastward (tesseral harmonic drive at 42.5 degE) |
| Mean lon at day 1.5 | 42.5338 degE (east trigger crossed) |
| Instantaneous lon at burn | 42.5287 degE |

The Phase 0 coast is entirely within the ±0.05 deg deadband. The eastward drift is gradual — approximately 0.022 deg/day mean longitude gain during the pre-burn phase.

7.2 Burn 1 (Day 1.5, 11 s Prograde)

| Quantity | Value |
| --- | --- |
| Epoch day | 1.5001 |
| Pre-burn SMA | 42163.8000 km (-0.37 km below synchronous) |
| Burn duration | 11 seconds prograde |
| Post-burn SMA | 42167.2000 km (+3.03 km above synchronous) |
| Delta SMA | +3.4000 km |
| Fuel consumed | 0.1020 kg |
| Post-burn longitude | 42.5287 degE (unchanged — burn is ~0 deg arc at GEO) |
| Drift state after burn | Westward (SMA > synchronous -> period > sidereal day) |

7.3 Phase 1 — Westward Coast (Day 1.5 to Day 16.5)

After Burn 1 raises SMA to 42167.2 km, the satellite enters a 15-day westward coast arc:

| Day | SMA (km) | Mean Lon (degE) | Position relative to slot |
| --- | --- | --- | --- |
| 1.5 (post-Burn 1) | 42167.2000 | 42.5338 | 0.034 deg east of centre |
| 3.0 | 42167.1000 | 42.5189 | 0.019 deg east of centre |
| 6.0 | 42166.7000 | 42.4954 | 0.005 deg west of centre |
| 9.0 | 42166.3000 | 42.4873 | ARC BOTTOM: 0.013 deg west of centre |
| 12.0 | 42165.9000 | 42.4947 | 0.005 deg west of centre |
| 13.0 | 42165.8000 | 42.5006 | Crosses slot centre eastward |
| 15.0 | 42165.5000 | 42.5175 | 0.018 deg east of centre |
| 16.0 | 42165.4000 | 42.5285 | 0.029 deg east of centre |
| 16.5 (pre-Burn 2) | 42165.2000 | 42.5328 | 0.033 deg east — east trigger |

The arc bottom at 42.4873 degE mean longitude (day 9.0) is 0.013 deg west of the slot centre and 0.023 deg above the 42.45 degE west limit — excellent compliance margin. The satellite then recovers eastward, crossing the slot centre at day 13 and reaching the east trigger at day 16.5.

7.4 Burn 2 (Day 16.5, 7 s Prograde, SMA-Matched)

| Quantity | Value |
| --- | --- |
| Epoch day | 16.5002 |
| Pre-burn SMA | 42165.2000 km (+1.03 km above synchronous) |
| Pre-burn fuel | 799.8980 kg (0.1020 kg used by Burn 1) |
| Required delta SMA | 42167.2000 - 42165.2000 = 2.0000 km |
| Formula duration | 2.0000 / 0.30909 = 6.471 s -> rounded UP to 7 s |
| Burn duration applied | 7 seconds prograde |
| Post-burn SMA | 42167.3000 km (+3.13 km above synchronous, +0.1 km overshoot) |
| Actual delta SMA | +2.1000 km (vs. 2.0 km target — overshoot from 7 s vs 6.47 s) |
| Fuel consumed | 0.0650 kg |
| Drift state after burn | Westward (SMA > synchronous) |

7.5 Phase 2 — Second Westward Arc (Day 16.5 to Day 33.5)

The second arc mirrors Phase 1 with a slightly deeper arc bottom due to the 0.1 km SMA overshoot from Burn 2:

| Day | Mean Lon (degE) | Notes |
| --- | --- | --- |
| 16.5 (post-Burn 2) | 42.5328 | East trigger — Burn 2 just fired |
| 20.0 | 42.5050 | Westward, approaching bottom |
| 25.3 | 42.4721 | ARC BOTTOM — 0.028 deg west of centre |
| 28.0 | 42.4870 | Recovering eastward |
| 31.0 | 42.5100 | Crossing slot centre area |
| 33.5 (end) | 42.5342 | East trigger re-crossed — where Burn 3 would fire |

The Phase 2 arc bottom (42.472 degE at day 25.3) is slightly deeper than Phase 1 (42.487 degE at day 9.0) — a 15 mdeg difference attributable to the 0.1 km SMA overshoot from Burn 2. The arc remains fully compliant: the minimum mean longitude (42.472 degE) is 0.022 deg above the 42.45 degE west limit.

The scenario ends at day 33.5, where the east trigger is crossed again — 17.0 days after Burn 2. This is where Burn 3 would fire in a continuing operational cycle. The cycle length is approximately 15–17 days depending on post-burn SMA accuracy.

8. Compliance Analysis

8.1 Instantaneous vs. Mean Longitude

GEO station-keeping compliance is evaluated on the 24-hour rolling mean longitude, not on the instantaneous longitude. The instantaneous longitude oscillates with a ~24-hour period (diurnal oscillation) due to residual eccentricity and inclination. For this scenario:

- Instantaneous longitude range: 42.420 degE to 42.582 degE (amplitude +/-0.081 deg — exceeds deadband limits)

- 24-hr mean longitude range: 42.472 degE to 42.534 degE (amplitude +/-0.031 deg — well inside deadband)

The large instantaneous range is a visual artefact, not a compliance violation. Real GEO operators monitor mean longitude derived from ranging data and telemetry — the diurnal oscillation is a known, predictable geometric effect that averages out over 24 hours.

8.2 Full Compliance Table

| Metric | Value | Limit | Margin | Result |
| --- | --- | --- | --- | --- |
| Min mean longitude | 42.4721 degE | >=42.45 degE | +0.0221 deg | PASS |
| Max mean longitude | 42.5342 degE | <=42.55 degE | +0.0158 deg | PASS |
| Min instantaneous lon | 42.4203 degE | (diurnal — not compliance metric) | N/A | N/A |
| Max instantaneous lon | 42.5822 degE | (diurnal — not compliance metric) | N/A | N/A |
| Deadband violations (mean lon) | 0 events | 0 allowed | Perfect | PASS |
| EWSK cycle period | 15.0 days | Consistent with ops history | On spec | PASS |
| Fuel used (2 burns) | 0.167 kg | <<1 kg expected | Nominal | PASS |
| Slot assignment | 42.5 degE (unchanged) | ITU filed — never change | Maintained | PASS |

RESULT: SK ARC COMPLIANT — Full 33.5-Day Cycle Within Deadband

9. Fuel Budget Analysis

| Item | Value |
| --- | --- |
| Fuel mass at epoch | 800.0000 kg |
| Fuel consumed — Burn 1 (11 s) | 0.1020 kg |
| Fuel consumed — Burn 2 (7 s) | 0.0650 kg |
| Total fuel consumed (2 burns) | 0.1670 kg |
| Fuel remaining at end | 799.8330 kg |
| Percentage consumed | 0.021% of total |
| Burns per year (15-day cycle) | ~24 burns/year |
| Projected annual EWSK fuel (linear) | ~2.0 kg/year |
| Satellite life (30-year GEO) | ~60 kg EWSK fuel total |

The fuel consumption figures are consistent with real GEO EWSK budgets: East-West station-keeping typically consumes 1-3 kg/year for a standard GEO satellite, compared to ~50 m/s/year (>>100 kg/year) for North-South station-keeping (inclination control). EWSK is the less fuel-intensive of the two principal GEO maintenance activities.

Note on burn-rate efficiency: Burn 2 uses only 0.065 kg vs. Burn 1's 0.102 kg. This is not because Burn 2 is less efficient — it is because Burn 2 is shorter (7 s vs. 11 s). The mass flow rate (dm/dt) is the same for both burns; the fuel difference directly reflects the burn duration ratio (7/11 = 0.636; 0.065/0.102 = 0.637).

10. Key Lessons and Engineering Insights

10.1 SMA-Matched Burns — The Core Insight

Lesson: EWSK burns must target the same post-burn SMA, not apply the same burn duration. The pre-burn SMA changes with every coast because the satellite's SMA decays back toward synchronous during the westward coast. Applying a fixed-duration burn from an elevated pre-burn SMA would over-raise the post-burn SMA, deepening the westward arc and potentially causing a west-limit violation.

Generalisation: This principle applies to every GEO satellite performing EWSK. The burn rate constant (dSMA/dt_burn) can be derived from any single burn; the SMA target is established empirically from a proven-compliant run; subsequent burns use the formula T_burn = (SMA_target - SMA_pre) / (dSMA/dt_burn) with rounding up.

10.2 SMA Decays During Westward Coast

Lesson: When the satellite is above synchronous SMA (westward drift), tesseral harmonics pull the SMA BACK toward synchronous — the SMA decays, not grows. This is counterintuitive but physically correct: the tesseral harmonic acts as a restoring force toward the stable equilibrium (75 degE or 255 degE), and at 42.5 degE this manifests as SMA decay during westward drift. The decay rate measured from 09b data is -0.133 km/day.

10.3 Burn Epoch Must be Data-Driven

Lesson: EWSK burn epochs cannot be set on a fixed calendar — they are triggered by the satellite's longitude reaching the operational east trigger. The 15-day cycle observed in 09b is an emergent property of the dynamics, not a design parameter. In practice, ground controllers monitor the mean longitude trend and command the burn when the trigger is anticipated within the next pass window.

10.4 Rounding Burn Duration — Always Up

Lesson: When the formula gives a non-integer burn duration, always round UP. The reason: an overshoot of SMA_target (too much delta-SMA) deepens the westward arc slightly, delaying the next east trigger crossing. An undershoot (too little delta-SMA) means the satellite returns to the east trigger sooner and may breach the east limit before the next planned burn. Overshoot is the safer error.

10.5 Compliance Evaluated on 24-hr Mean Longitude

Lesson: The instantaneous longitude oscillates ±0.08 deg (diurnal, from ECC and INC). This amplitude exceeds the ±0.05 deg deadband at some phases. Evaluating compliance on instantaneous longitude would give false violations and lead to unnecessary burns. ITU and real operators evaluate the 24-hour mean longitude — the physically meaningful quantity for GEO slot compliance.

10.6 Stopping Conditions Must Use ElapsedDays/ElapsedSecs

Lesson: GMAT longitude-based stopping conditions would trigger on instantaneous longitude crossings, not mean longitude crossings — causing premature or erratic burns. The correct GMAT implementation coasts for the known elapsed time derived from post-processing of the prior run's data.

10.7 GMAT R2022a Constraints

- Every resource property line requires the GMAT prefix (e.g., GMAT SC.SMA = 42164.1696)

- Report file path must be an absolute Windows path (C:\...\report.txt)

- Scripts must be strict 7-bit ASCII — no degree signs, no +/- Unicode, no em-dashes or arrows in comments

- Non-ASCII characters cause Interpreter Exception before the script parses

- Column layout in ReportFile output: UTCGregorian (4 parts), Longitude, ECC, SMA, TotalMass, FuelMass

11. Output Files

| File | Description |
| --- | --- |
| 09b_geo_ewsk.script | GMAT R2022a script — mission sequence, all resources, report config |
| 09b_geo_ewsk_report.txt | GMAT output — 3,661 rows, day 0.0 to 33.5, all 6 columns |
| plot_ewsk_09b.py | Python post-processor — XY plot generation from report data |
| 09b_ewsk_XYplot.png | Station-keeping plot — instantaneous lon, 24-hr mean, burn markers, compliance badge |

All files located in: C:\Users\Admin\GMAT_Tutorials\GMAT_portfolio\09_scenario_EWSK\scenairo_09b\

Note: The folder name scenairo_09b (typo: "scenairo" rather than "scenario") is preserved exactly in all paths — changing it would break the absolute path in the GMAT script ReportFile.Filename.

12. Operational Significance

12.1 NigComSat-1R Operational Parallel

This scenario directly models the EWSK operations Stephen O. Ogodo performed at NigComSat for 15 years. The 15-day cycle, 42.53 degE east trigger, single prograde burn, and SMA-based duration calculation are all consistent with real NigComSat-1R operational practice. The simulation validates the physics that governed the satellite's real daily operations.

12.2 Generalisation to Other GEO Satellites

The generalised burn duration formula derived in Section 6 applies directly to any GEO satellite performing EWSK:

- Derive dSMA/dt_burn from the first burn (or from thrust/mass/Isp parameters)

- Establish SMA_target from a proven-compliant run or from the required arc geometry

- For each subsequent burn, read SMA_pre from the most recent ranging data

- Apply T_burn = (SMA_target - SMA_pre) / (dSMA/dt_burn), round up to integer seconds

- Upload as a finite-burn command with the derived duration

This is a closed-form operational procedure that does not require a GMAT run for each cycle — only an initial calibration run to establish dSMA/dt_burn and SMA_target. Subsequent burns are calculated analytically from ranging telemetry.

12.3 Connection to Scenario 10 (North-South SK)

The planned Scenario 10 (NSSK) will complement this scenario. EWSK (this scenario) controls longitude drift using prograde burns that change SMA. NSSK controls inclination growth (~0.75 deg/year from lunisolar perturbations) using out-of-plane normal burns. NSSK is far more fuel-expensive (~50 m/s/year vs. ~1 m/s/year for EWSK) and is the dominant factor in GEO satellite operational lifetime. Together, Scenarios 09 and 10 cover the complete GEO station-keeping discipline.

13. Summary: Key Numbers

| Category | Parameter | Value |
| --- | --- | --- |
| Slot | Assigned longitude | 42.5 degE (ITU filed — NigComSat-1R) |
| Slot | Deadband | +/-0.05 deg (42.45 to 42.55 degE) |
| Slot | East operational trigger | 42.53 degE (mean lon) |
| Slot | West operational trigger | 42.47 degE (mean lon) |
| Physics | Synchronous SMA | 42164.1696 km |
| Physics | Burn rate constant | 0.30909 km/s (from Burn 1: 3.4/11) |
| Physics | SMA target (post-burn) | 42167.2000 km (= sync + 3.03 km) |
| Physics | Coast SMA decay rate | -0.133 km/day |
| Burn 1 | Epoch | Day 1.5 (east trigger) |
| Burn 1 | Duration | 11 s prograde |
| Burn 1 | Delta SMA | +3.4 km (42163.8 -> 42167.2 km) |
| Burn 1 | Fuel | 0.102 kg |
| Burn 2 | Epoch | Day 16.5 (east trigger, 15 days after Burn 1) |
| Burn 2 | Pre-burn SMA | 42165.2 km (decayed from 42167.2 over 15 days) |
| Burn 2 | Duration formula | (42167.2 - 42165.2) / 0.309 = 6.47 s -> 7 s |
| Burn 2 | Duration applied | 7 s prograde |
| Burn 2 | Delta SMA | +2.1 km (42165.2 -> 42167.3 km) |
| Burn 2 | Fuel | 0.065 kg |
| Results | Min mean longitude | 42.472 degE (Phase 2 arc bottom, day 25.3) |
| Results | Max mean longitude | 42.534 degE (at east trigger crossings) |
| Results | Total fuel | 0.167 kg (99.98% remaining) |
| Results | Cycle period | 15.0 days (Burn 1 to Burn 2) |
| Results | Scenario duration | 33.5 days (2 complete arcs) |
| Results | Compliance | PASS — SK ARC COMPLIANT (green badge) |

End of Scenario 09b Documentation

GMAT Astrodynamics Portfolio  |  Stephen O. Ogodo  |  October 2026