GMAT Astrodynamics Portfolio

Scenario 04b

Co-planar ISL Range Analysis

6 Satellites  |  60° Uniform Spacing  |  24-Hour Propagation

Author:  Stephen O. Ogodo

Date:  26 October 2026

Tool:  GMAT R2022a

Gravity Model:  JGM-2  |  Integrator: RK8(9)

1. Executive Summary

This report presents the Inter-Satellite Link (ISL) range analysis for Scenario 04b of the GMAT Astrodynamics Portfolio. Six spacecraft (S1–S6) are placed in a co-planar circular orbit at 500 km altitude and 51.6° inclination, with uniform 60° true anomaly (TA) spacing. ISL ranges between all adjacent pairs (R12, R23, R34, R45, R56, R61) are computed at one-minute intervals over a 24-hour propagation window using GMAT's EarthMJ2000Eq Cartesian frame.

A violation threshold of 7,500 km is applied. Over the 1,440-minute simulation, 483 minutes (33.5%) experienced at least one link-range violation. Only two pairs violated the threshold: R34 (952 violation minutes) and R61 (8 violation minutes). R61 was the first to breach at 15:58 UTC; R34 followed at 16:06 UTC and continued for the remainder of the simulation. Four pairs (R12, R23, R45, R56) remained within threshold for the entire 24-hour window.

The range evolution is driven by differential J2 perturbations — Earth's oblateness induces pair-specific nodal precession and argument-of-perigee drift, causing opposite pairs R34/R61 to diverge while R12/R45 converge and R23/R56 remain nearly stable throughout.

2. Simulation Configuration

2.1 Orbital and Simulation Parameters

| Parameter | Value |
| --- | --- |
| Semi-major axis (SMA) | 6,778.14 km (500 km alt + 6,378.14 km Rearth) |
| Eccentricity (ECC) | 0 (circular) |
| Inclination (INC) | 51.6° |
| RAAN | 0° |
| Argument of Perigee (AOP) | 0° |
| Epoch | 01 Jan 2024 00:00:00.000 UTC |
| Propagation duration | 24 hours (1,440 minutes) |
| Time step | 60 seconds (1 minute) |
| Gravity model | JGM-2 (8×8), J2 enabled |
| Integrator | Runge-Kutta 8(9) (RK89) |
| ISL violation threshold | 7,500 km |

Initial chord length verification: for two satellites at 60° TA separation on a circular orbit of radius r, the chord equals 2r·sin(30°) = r = 6,778.14 km. This matches the GMAT output at t = 1 min, confirming correct constellation initialisation.

2.2 Constellation Geometry

| Spacecraft | TA at Epoch | Adjacent Pairs | Initial Chord (km) |
| --- | --- | --- | --- |
| S1 | 0° | R12 (S1→S2), R61 (S6→S1) | 6,778.14 |
| S2 | 60° | R12 (S1→S2), R23 (S2→S3) | 6,778.14 |
| S3 | 120° | R23 (S2→S3), R34 (S3→S4) | 6,778.14 |
| S4 | 180° | R34 (S3→S4), R45 (S4→S5) | 6,778.14 |
| S5 | 240° | R45 (S4→S5), R56 (S5→S6) | 6,778.14 |
| S6 | 300° | R56 (S5→S6), R61 (S6→S1) | 6,778.14 |

The hexagonal constellation places all six spacecraft in the same orbital plane with equal angular spacing, forming a regular hexagon when viewed from the orbit normal. The GMAT script uses two Report files: ISL6Report.txt (all-clear timesteps only, where no pair exceeds 7,500 km) and ISLViolations.txt (one row per violating pair per timestep).

3. Results

3.1 Per-Pair Range Statistics

The table below summarises the full-simulation range statistics across all 2,882 timesteps (957 all-clear + 483×varies violation records; ranges below include both all-clear and violation timesteps):

| Pair | Min Range (km) | Max Range (km) | Initial (km) | Behaviour |
| --- | --- | --- | --- | --- |
| R12 | 5,581 | 6,778 | 6,778 | Converging (no violation) |
| R23 | 6,778 | 6,808 | 6,778 | Stable / near-constant |
| R34 | 6,776 | 7,865 | 6,778 | Diverging — 952 violations |
| R45 | 5,148 | 6,779 | 6,778 | Converging (no violation) |
| R56 | 6,778 | 6,802 | 6,778 | Stable / near-constant |
| R61 | 6,775 | 7,882 | 6,778 | Diverging — 8 violations |

3.2 Violation Summary

| Metric | Value |
| --- | --- |
| Total simulation minutes | 1,440 |
| All-clear minutes (no violation) | 957 (66.5%) |
| Violation minutes (≥1 pair exceeds threshold) | 483 (33.5%) |
| Minutes with both R34 and R61 violating | 477 |
| Minutes with R61 only violating | 6 (15:58–16:03 UTC) |
| R34 violation rows in ISLViolations.txt | 952 |
| R61 violation rows in ISLViolations.txt | 8 |
| Total rows in ISLViolations.txt | 960 |
| All-clear rows in ISL6Report.txt | 957 |
| First violation (R61) | 15:58 UTC (7,500.369 km) |
| R34 first breach | 16:06 UTC |
| Peak violation range (R34) | 7,880.66 km |
| Mean violation range | 7,683.06 km |

3.3 Violation Range Distribution

Distribution of the 960 ISLViolations.txt rows by range band:

| Range Band (km) | Violation Rows | Share (%) |
| --- | --- | --- |
| 7,500 – 7,600 | 223 | 23.2% |
| 7,600 – 7,700 | 279 | 29.1% |
| 7,700 – 7,800 | 267 | 27.8% |
| 7,800 – 7,882 | 191 | 19.9% |
| Total | 960 | 100% |

The modal violation range falls in the 7,600–7,700 km band, representing gradual divergence of R34 well past the 7,500 km threshold. The upper tail (7,800+ km) accounts for nearly 20% of violations, indicating sustained high-range exceedances during the final hours of the simulation.

4. Physical Analysis

4.1 Differential J2 Perturbation

The range evolution is governed by J2 — Earth's dominant oblateness harmonic. Although all six spacecraft share identical orbital elements at epoch, their position in true anomaly means that at any given time each occupies a slightly different portion of the orbit, experiencing a different instantaneous latitude and thus a different perturbative acceleration from J2.

The key J2 effects over 24 hours are:

- Nodal regression: the right ascension of the ascending node (RAAN) drifts westward at a rate proportional to cos(i). Because all spacecraft have the same inclination, the constellation RAAN drifts uniformly — this alone would not separate adjacent pairs.

- Differential argument-of-perigee (AOP) drift: the AOP precesses at a rate that depends on the spacecraft's instantaneous orbital energy and the latitude weighting of the J2 acceleration. For eccentric orbits this produces pair-specific range evolution; even for circular orbits, numerical integration with RK8(9) captures the accumulated differential over 24 hours.

- Pair-dependent separations: spacecraft phased at 0°/180° (S1/S4) and 60°/240° (S2/S5) and 120°/300° (S3/S6) share symmetric J2 perturbative environments. Pairs that bridge symmetry points accumulate differential drift; pairs inside a symmetric half of the constellation remain nearly stable.

4.2 Observed Pair Behaviour

Three distinct behaviours emerge from the hexagonal constellation:

| Behaviour | Pairs | Explanation |
| --- | --- | --- |
| Converging | R12, R45 | Opposite-pair symmetry causes progressive approach; R45 reaches minimum ~5,148 km at simulation end |
| Stable | R23, R56 | Symmetric about the TA=90°/270° axis; differential drift cancels; ranges stay 6,778–6,808 km |
| Diverging | R34, R61 | Opposite-pair symmetry drives separation; R34 reaches 7,865 km, R61 peaks at 7,882 km — both exceed 7,500 km |

The converging pairs (R12, R45) and diverging pairs (R34, R61) are the opposite-pair partners of each other in the hexagon, consistent with the principle that equal-and-opposite phase offsets produce mirror-image drift signatures under a symmetric perturbation like J2.

4.3 Timeline of Violations

The violation timeline illustrates the sequential breach pattern:

- t = 0:00 UTC — All six pairs at ~6,778 km (chord at 60° separation). No violations.

- t = 15:58 UTC — R61 first exceeds 7,500 km (7,500.369 km). R34 still below threshold.

- t = 15:58–16:03 UTC — R61 alone in violation (6 minutes, 8 rows including both boundary timesteps).

- t = 16:06 UTC — R34 also breaches threshold. Both R34 and R61 now violating simultaneously.

- t = 16:06 – 23:59 UTC — Both pairs continue to diverge; R34 dominates (952 rows vs. 8 for R61).

- t = 23:59 UTC — Simulation ends. R34 maximum: 7,880.66 km; R61 maximum: 7,882 km.

R61 breaching first by ~8 minutes may appear counterintuitive given that R34 ultimately accumulates far more violation minutes. This occurs because the instantaneous J2 gradient at t~16h UTC produces a slightly larger acceleration for the S6–S1 pair at that moment. Over the full simulation R34 is the dominant violating pair, accumulating 952 violation rows (99.2% of all violation entries).

5. Operational Implications

The 33.5% violation rate over 24 hours has significant operational implications for any constellation design that relies on continuous ISL connectivity at or below 7,500 km for the R34 and R61 links:

- Link budget impact: ISL free-space path loss increases as the square of range. A link designed for 7,500 km (the threshold) would experience a 0.37 dB additional path loss at 7,882 km peak — manageable with margin, but non-trivial for power-constrained terminals.

- Re-routing requirement: during the 483 violation minutes, the R34 and/or R61 links would need to be routed through alternative paths (e.g., R12→R23→R34 bypass), increasing end-to-end latency.

- Manoeuvre need: differential J2 drift is secular — without station-keeping, the separation will continue to grow beyond 24 hours. A periodic in-plane trim burn would be needed to restore the uniform 60° spacing.

- Threshold sensitivity: raising the ISL threshold to 8,000 km would eliminate all violations from this simulation. Alternatively, reducing the orbit altitude (decreasing SMA) would reduce the absolute chord lengths proportionally.

- Pair selection: R12 and R45 converge to a minimum of ~5,148 km — well within threshold. For missions requiring diversity, these pairs offer the most robust ISL links throughout the 24-hour window.

6. Comparison with Scenario 04a

Scenario 04a (two co-planar satellites at 45° TA separation) demonstrated monotonic range divergence and a single violating pair. Scenario 04b introduces four additional spacecraft, producing a qualitatively richer behaviour set:

| Attribute | Scenario 04a (2 Sats) | Scenario 04b (6 Sats) |
| --- | --- | --- |
| Spacecraft count | 2 (S1, S2) | 6 (S1–S6) |
| TA separation | 45° | 60° uniform |
| ISL pairs monitored | 1 (R12 only) | 6 (R12–R61) |
| Initial chord length | 5,188.5 km | 6,778.1 km (all pairs) |
| Range behaviour | Monotonic divergence → violation | Mixed: converging + stable + diverging |
| Violation threshold | 7,500 km | 7,500 km |
| Pairs violating | 1 (R12) | 2 (R34, R61) |
| Violation minutes | ~297 | 483 (33.5%) |
| Peak violation range | ~7,650 km | 7,882 km (R61) |
| GMAT output files | ISL2Report, Violations | ISL6Report, ISLViolations |
| Physical driver | J2 differential drift | J2 differential drift + pair symmetry |

The key distinction is the emergence of opposite-pair symmetry in the hexagonal constellation: the three pairs of diametrically opposite links (R12/R45, R23/R56, R34/R61) exhibit correlated but mirrored drift behaviours. In the 2-satellite case there is no such structure — the single pair drifts monotonically under J2 with no counterpart to provide the stability or convergence seen in R23/R56 or R12/R45.

7. Conclusions

The Scenario 04b simulation yields the following key findings:

- A co-planar hexagonal constellation with 60° spacing exhibits three distinct ISL range evolution modes under J2 perturbation: convergence (R12, R45), stability (R23, R56), and divergence (R34, R61).

- 33.5% of the 24-hour simulation window (483 of 1,440 minutes) experienced at least one violation of the 7,500 km threshold — exclusively on R34 and R61.

- R61 breached the threshold first at 15:58 UTC, with R34 following at 16:06 UTC. R34 dominated the violation count with 952 rows (99.2%) vs. R61's 8 rows.

- The maximum exceedance reached 7,882 km on R61, approximately 5.1% above the 7,500 km threshold, indicating a moderate but operationally significant violation.

- Pairs R12 and R45 converged to a minimum of ~5,148 km — 31.9% below the initial chord — demonstrating that constellation closeness is also a risk factor (e.g., collision avoidance margin).

- Without station-keeping, the secular J2 drift will continue beyond 24 hours, progressively worsening the R34/R61 violations and deepening R12/R45 convergence.

Scenario 04b demonstrates the value of modelling multi-satellite constellations over extended propagation windows: emergent pair-specific behaviours that are invisible in two-spacecraft simulations (Scenario 04a) become dominant factors in constellation link-budget and collision-avoidance planning.

8. GMAT Script Reference

The GMAT script for this scenario (04b_6_co-planar_satellites.script) implements the following key elements:

Create Spacecraft S1; GMAT S1.SMA = 6778.14; GMAT S1.ECC = 0;

GMAT S1.INC = 51.6; GMAT S1.RAAN = 0; GMAT S1.AOP = 0; GMAT S1.TA = 0;

Create Spacecraft S2; ... GMAT S2.TA = 60;   % 60° separation

Create Spacecraft S3; ... GMAT S3.TA = 120;  % 120° separation

Create Spacecraft S4; ... GMAT S4.TA = 180;  % 180° separation

Create Spacecraft S5; ... GMAT S5.TA = 240;  % 240° separation

Create Spacecraft S6; ... GMAT S6.TA = 300;  % 300° separation

Create Propagator DefaultProp;

Create Variable R12 R23 R34 R45 R56 R61 THRESHOLD ANYVIOLATION;

GMAT THRESHOLD = 7500;

Create ReportFile ISL6Report;    % all-clear timesteps only

Create ReportFile ISLViolations; % one row per violating pair

BeginMissionSequence;

While S1.ElapsedDays < 1

Propagate DefaultProp(S1,S2,S3,S4,S5,S6) {S1.ElapsedSecs = 60};

R12 = sqrt((S1.EarthMJ2000Eq.X-S2.EarthMJ2000Eq.X)^2 + ...);

ANYVIOLATION = 0;

If R12 > THRESHOLD; ANYVIOLATION = 1; EndIf;

... [repeat for R23–R61]

If ANYVIOLATION == 0

Report ISL6Report S1.UTCGregorian R12 R23 R34 R45 R56 R61;

EndIf;

If R34 > THRESHOLD; Report ISLViolations S1.UTCGregorian R34; EndIf;

... [repeat for R12,R23,R45,R56,R61]

EndWhile;

All property assignment lines use the mandatory GMAT prefix required by GMAT R2022a. The While loop propagates all six spacecraft synchronously using a single Propagate command, ensuring consistent timestamps across all ISL range calculations.