GMAT Astrodynamics Portfolio

Scenario 05

Inter-Plane ISL Range Analysis

2 Satellites  |  90° RAAN Separation  |  Cross-Plane Link Windows

Author:  Stephen O. Ogodo

Date:  26 October 2026

Tool:  GMAT R2022a

Gravity Model:  JGM-2  |  Integrator: RK8(9)

1. Executive Summary

This report presents the Inter-Satellite Link (ISL) range analysis for Scenario 05 of the GMAT Astrodynamics Portfolio. Two spacecraft — SatA (Plane 1, RAAN = 0°) and SatB (Plane 2, RAAN = 90°) — are placed in identical circular orbits at 500 km altitude and 51.6° inclination, separated only by a 90° difference in their right ascension of the ascending node (RAAN). The cross-plane ISL range is computed at one-minute intervals over a 24-hour propagation window.

A link-viable threshold of 6,500 km is applied. This is more stringent than the 7,500 km threshold used in Scenarios 04a/04b, reflecting the additional margin required for cross-plane links due to higher range variability and Doppler effects. Over the 1,440-minute simulation, 31 ISL windows were identified — each lasting 10–11 minutes and recurring approximately every 46 minutes, driven by the ~92.6 min orbital period. In total, 323 minutes (22.4%) of the simulation fell within a link-viable window.

The cross-plane geometry produces a fundamentally different range profile compared to co-planar scenarios: the range oscillates between a minimum of ~5,953 km (close approach at orbital plane intersection) and a maximum of ~9,586 km (maximum separation at half-orbit offset). The periodicity is governed by both spacecraft's orbital period and the 90° nodal offset.

2. Simulation Configuration

2.1 Spacecraft Orbital Elements

| Parameter | SatA (Plane 1) | SatB (Plane 2) |
| --- | --- | --- |
| Semi-major axis (SMA) | 6,778.14 km | 6,778.14 km |
| Eccentricity (ECC) | 0.0001 (near-circular) | 0.0001 (near-circular) |
| Inclination (INC) | 51.6° | 51.6° |
| RAAN | 0° (Plane 1) | 90° (Plane 2) |
| Argument of Perigee (AOP) | 0° | 0° |
| True Anomaly (TA) | 0° | 0° |
| Epoch | 01 Jan 2024 00:00:00.000 UTC | 01 Jan 2024 00:00:00.000 UTC |

The key differentiator from Scenarios 04a/04b is the RAAN offset: both satellites are in identical orbits but in perpendicular planes (RAAN = 0° vs RAAN = 90°). They share the same altitude, inclination, and TA at epoch — only their ascending node longitudes differ. This creates a cross-plane geometry where the satellites periodically approach each other at or near the intersection of their orbital planes.

2.2 Simulation Parameters

| Parameter | Value |
| --- | --- |
| Propagation duration | 24 hours (1,440 minutes) |
| Time step | 60 seconds (1 minute) |
| Gravity model | JGM-2 (8×8), J2 enabled |
| Integrator | Runge-Kutta 8(9) (RK89) |
| ISL range frame | EarthMJ2000Eq (Cartesian) |
| Link-viable threshold | 6,500 km |
| Orbital period (computed) | ≈ 92.6 minutes |

Two output files are generated: InterplaneISLReport.txt logs the full Cartesian state of both spacecraft and the computed ISL range at every timestep (1,440 rows); InterplaneISLWindows.txt is a companion file listing timesteps where the range was below the 6,500 km threshold. The ISL range is computed analytically from the EarthMJ2000Eq Cartesian positions:

ISL_Range = sqrt((SatA.X - SatB.X)² + (SatA.Y - SatB.Y)² + (SatA.Z - SatB.Z)²)

3. Results

3.1 Summary Statistics

| Metric | Value |
| --- | --- |
| Total simulation minutes | 1,440 |
| Link-viable minutes (ISL_Range ≤ 6,500 km) | 323 (22.4%) |
| Above-threshold minutes (ISL_Range > 6,500 km) | 1,117 (77.6%) |
| Number of ISL windows | 31 |
| Window duration (per window) | 10–11 minutes |
| Inter-window gap (approx.) | ~46 minutes |
| Minimum ISL range (global) | 5,952.9 km (08:01 UTC, W11) |
| Maximum ISL range (global) | 9,585.6 km |
| Mean ISL range (all 1,440 min) | 7,879.7 km |
| Mean ISL range (link-viable only) | 6,142 km |
| First window start | 00:19 UTC |
| Last window end | 23:35 UTC |

3.2 Range Distribution

Distribution of ISL range across all 1,440 simulation minutes:

| Range Band (km) | Minutes | Share (%) |
| --- | --- | --- |
| ≤ 6,500 (link-viable) | 323 | 22.4% |
| 6,500 – 7,500 | 274 | 19.0% |
| 7,500 – 8,500 | 263 | 18.3% |
| 8,500 – 9,000 | 164 | 11.4% |
| > 9,000 | 416 | 28.9% |
| Total | 1,440 | 100% |

The bimodal-like distribution — with a large fraction both below 6,500 km (link-viable) and above 9,000 km (maximum separation) — reflects the sinusoidal range profile of a cross-plane geometry: the range spends time near both extremes as it oscillates between close approach and maximum separation.

3.3 ISL Link Windows

31 link windows were identified over the 24-hour simulation (01 Jan 2024). Each window represents a continuous interval where the cross-plane ISL range remained below the 6,500 km threshold:

| Window | Start (UTC) | End (UTC) | Duration (min) | Min Range (km) |
| --- | --- | --- | --- | --- |
| W01 | 00:19 | 00:30 | 11 | 5,953.1 |
| W02 | 01:06 | 01:16 | 10 | 5,954.7 |
| W03 | 01:52 | 02:02 | 10 | 5,958.8 |
| W04 | 02:38 | 02:48 | 10 | 5,957.3 |
| W05 | 03:24 | 03:35 | 11 | 5,954.7 |
| W06 | 04:10 | 04:21 | 11 | 5,954.2 |
| W07 | 04:56 | 05:07 | 11 | 5,955.2 |
| W08 | 05:43 | 05:53 | 10 | 5,956.7 |
| W09 | 06:29 | 06:39 | 10 | 5,957.4 |
| W10 | 07:15 | 07:25 | 10 | 5,955.9 |
| W11 | 08:01 | 08:12 | 11 | 5,952.9 ← global min |
| W12 | 08:47 | 08:58 | 11 | 5,954.1 |
| W13 | 09:34 | 09:44 | 10 | 5,954.8 |
| W14 | 10:20 | 10:30 | 10 | 5,958.4 |
| W15 | 11:06 | 11:16 | 10 | 5,955.3 |
| W16 | 11:52 | 12:03 | 11 | 5,955.6 |
| W17 | 12:38 | 12:49 | 11 | 5,953.6 |
| W18 | 13:24 | 13:35 | 11 | 5,955.9 |
| W19 | 14:11 | 14:21 | 10 | 5,957.7 |
| W20 | 14:57 | 15:07 | 10 | 5,960.7 |
| W21 | 15:43 | 15:53 | 10 | 5,955.8 |
| W22 | 16:29 | 16:40 | 11 | 5,956.5 |
| W23 | 17:15 | 17:26 | 11 | 5,955.4 |
| W24 | 18:02 | 18:12 | 10 | 5,957.5 |
| W25 | 18:48 | 18:58 | 10 | 5,960.3 |
| W26 | 19:34 | 19:44 | 10 | 5,958.3 |
| W27 | 20:20 | 20:31 | 11 | 5,955.1 |
| W28 | 21:06 | 21:17 | 11 | 5,954.7 |
| W29 | 21:53 | 22:03 | 10 | 5,955.9 |
| W30 | 22:39 | 22:49 | 10 | 5,957.6 |
| W31 | 23:25 | 23:35 | 10 | 5,959.0 |

Window W11 (08:01–08:12 UTC) contains the global range minimum of 5,952.9 km. Window durations alternate between 10 and 11 minutes, reflecting the integer truncation of the 92.6-minute orbital period at 1-minute timestep resolution. The last window (W31) ends at 23:35 UTC — indicating that a 32nd window would begin shortly after midnight, consistent with the ~46-minute periodicity.

4. Physical Analysis

4.1 Cross-Plane Geometry

When two satellites orbit in planes separated by a RAAN offset of 90°, their orbital planes intersect along the line of nodes — a great circle through the Earth's centre at the inclination angle of the orbit. For a 51.6° inclined orbit with RAAN separation of 90°, the two planes share two intersection points (ascending node of one ≈ the equatorial crossings of both planes).

A close approach (minimum range) occurs when both satellites are simultaneously near the same intersection point of their respective planes. Because the two satellites start at TA = 0° (ascending node) at epoch and have the same orbital period (~92.6 min), they pass through the nodal intersection at the same phase relative to each other. The angular velocity mismatch due to J2 differential drift is small over 24 hours, so the close-approach geometry repeats nearly periodically throughout the simulation.

4.2 Window Periodicity

The ~46-minute inter-window interval is approximately half the orbital period (92.6 / 2 = 46.3 min). This arises because two close approaches occur per orbital revolution — one at each of the two plane-intersection nodes (ascending and descending). The two satellites pass through both nodes during each orbit, producing two link windows per period:

- Node 1 (ascending intersection): both satellites near equatorial crossing → window opens

- Node 2 (descending intersection): ~46 min later, both near the opposite crossing → second window

- Next orbit: ~46 min later again → window repeats with similar geometry

The slight variation in inter-window spacing (46–47 minutes) results from the Earth's rotation beneath the orbit and from small differential perturbations (J2 nodal regression and AOP drift) accumulating over 24 hours. The 10/11-minute window duration variation reflects 1-minute timestep discretisation of the actual smooth approach and departure profile.

4.3 Range Profile Characteristics

The ISL range profile between cross-plane satellites is characteristically sinusoidal, unlike the monotonic divergence seen in some co-planar scenarios:

- Close approach (minimum): ~5,953–5,961 km. This is approximately equal to the diagonal chord across the orbital sphere at the angle subtended by the plane-intersection geometry. The minimum range is bounded below by 2 × SMA × sin(θ/2) where θ is the angular separation at closest approach.

- Maximum separation: ~9,586 km. This occurs when the two satellites are in opposite quarters of their respective orbital planes, maximising the 3D Cartesian distance across the 90° RAAN offset.

- Mean range: ~7,880 km across all 1,440 minutes — above the 6,500 km threshold, confirming that the link is unavailable for most of the orbit.

Compared to Scenario 04a (co-planar, 45° TA separation, monotonic divergence under J2), the cross-plane geometry produces a cyclic, opportunity-window model: the link is unavailable 77.6% of the time but becomes viable for predictable, well-defined 10–11 minute intervals recurring every ~46 minutes.

4.4 J2 Perturbation Effects

Both spacecraft experience the same J2 nodal regression rate (same i, same altitude), so differential J2 effects are minimal over 24 hours compared to the dominant geometric periodicity. The 90° RAAN offset is preserved throughout the simulation. Minor variations in window timing and duration are attributable to:

- Earth's rotation carrying the sub-satellite ground track westward between windows

- Higher-order perturbations (non-spherical harmonics beyond J2) captured by JGM-2

- 1-minute timestep discretisation producing ±1-minute window boundary uncertainty

5. Operational Implications

The cross-plane ISL window structure has significant implications for constellation design and inter-plane link scheduling:

- Predictable link schedule: the 31 windows are precisely timed and repeat every ~46 minutes. A constellation management system can pre-compute link windows for the full 24-hour operational day and schedule data-relay opportunities accordingly.

- Short contact windows: 10–11 minutes is sufficient for burst data transfers at moderate data rates (e.g., a 10 Mbps ISL terminal transfers 6–6.6 GB per window), but too short for continuous real-time relay without store-and-forward buffering.

- Range budget: the minimum link range (~5,953 km) is 8.4% below the 6,500 km threshold. An ISL link budget designed for 6,500 km would have ~1.1 dB of margin at closest approach — useful headroom for atmospheric or pointing losses.

- Doppler: cross-plane close approaches involve significant relative velocity components. At the nodal crossing the radial velocity between the two spacecraft can be substantial, requiring Doppler pre-compensation or frequency-agile ISL modems.

- Walker constellation extension: adding more planes (e.g., 6 planes at 15° RAAN spacing) would reduce inter-window gaps and increase total link-viable time. This scenario quantifies the 2-plane lower bound on cross-plane ISL availability.

- Comparison with co-planar (Scenario 04b): the 6-satellite co-planar constellation provides continuous ISL availability (66.5% all-clear in Scenario 04b) but within a single orbital plane. Cross-plane links (Scenario 05) provide inter-plane connectivity at the cost of intermittent availability.

6. Comparison with Prior Scenarios

| Attribute | 04a (2-sat co-plane) | 04b (6-sat co-plane) | 05 (2-sat cross-plane) |
| --- | --- | --- | --- |
| Satellites | 2 | 6 | 2 |
| TA separation | 45° | 60° | 0° (same) |
| RAAN separation | 0° (same plane) | 0° (same plane) | 90° |
| Threshold | 7,500 km | 7,500 km | 6,500 km |
| Range profile | Monotonic divergence | Mixed (converge / stable / diverge) | Sinusoidal (periodic windows) |
| Link availability | Continuous → violation | 66.5% all-clear | 22.4% viable |
| Window count | N/A | N/A | 31 windows |
| Min range | ~5,189 km initial | 5,148 km (R45) | 5,953 km |
| Max range | ~7,650 km | 7,882 km (R61) | 9,586 km |
| Physical driver | J2 differential drift | J2 + pair symmetry | Orbital geometry + J2 |

The key insight from the three ISL scenarios is the trade-off between co-planar and cross-plane link geometry: co-planar links (Scenarios 04a/04b) can provide higher availability within a single orbital plane but are vulnerable to J2-driven range divergence over time. Cross-plane links (Scenario 05) offer predictable, periodic close-approach windows that enable inter-plane connectivity at the cost of lower duty cycle (22.4% vs. 66.5%).

A full multi-plane Walker constellation would combine both link types: intra-plane co-planar links for continuous local connectivity, and cross-plane links for inter-plane data relay during the predictable indow intervals.

7. Conclusions

The Scenario 05 cross-plane ISL simulation yields the following key findings:

- Two LEO satellites in perpendicular orbital planes (RAAN = 0° and RAAN = 90°) produce a sinusoidal ISL range profile oscillating between 5,952.9 km (close approach) and 9,585.6 km (maximum separation) over a 24-hour period.

- 31 link-viable windows were identified, each lasting 10–11 minutes and recurring every ~46 minutes — approximately half the 92.6-minute orbital period, reflecting the two nodal-crossing opportunities per orbit.

- 22.4% of the simulation (323 of 1,440 minutes) fell within a link-viable window, compared to 66.5% all-clear availability in the 6-satellite co-planar Scenario 04b.

- The global minimum range of 5,952.9 km occurred at Window W11 (08:01–08:12 UTC), 8.4% below the 6,500 km threshold and providing positive link margin.

- Both spacecraft experience nearly identical J2 perturbations (same orbital elements except RAAN), so differential nodal drift is minimal and the window schedule remains stable across the 24-hour simulation.

- Scenario 05 establishes the baseline for inter-plane ISL feasibility in a two-plane LEO constellation and motivates the addition of more orbital planes (Walker constellation) to increase cross-plane link duty cycle above the 22.4% lower bound.

8. GMAT Script Reference

The script file 05_Cross_planar_2_Satellites.script implements the cross-plane ISL scenario with the following structure:

Create Spacecraft SatA; GMAT SatA.SMA = 6778.14; GMAT SatA.ECC = 0.0001;

GMAT SatA.INC = 51.6; GMAT SatA.RAAN = 0; GMAT SatA.AOP = 0; GMAT SatA.TA = 0;

Create Spacecraft SatB; GMAT SatB.SMA = 6778.14; GMAT SatB.ECC = 0.0001;

GMAT SatB.INC = 51.6; GMAT SatB.RAAN = 90; % <-- 90° RAAN offset

GMAT SatB.AOP = 0; GMAT SatB.TA = 0;

Create Variable ISL_Range THRESHOLD;

GMAT THRESHOLD = 6500;

Create ReportFile InterplaneISLReport;  % all timesteps

Create ReportFile InterplaneISLWindows; % timesteps below threshold

BeginMissionSequence;

Propagate DefaultProp(SatA, SatB) {SatA.ElapsedSecs = 60}; % seed step

ISL_Range = sqrt((SatA.EarthMJ2000Eq.X-SatB.EarthMJ2000Eq.X)^2 + ...);

While SatA.ElapsedDays < 1

Propagate DefaultProp(SatA, SatB) {SatA.ElapsedSecs = 60};

ISL_Range = sqrt((SatA.X-SatB.X)^2+(SatA.Y-SatB.Y)^2+(SatA.Z-SatB.Z)^2);

Report InterplaneISLReport SatA.UTCGregorian SatA.X SatA.Y SatA.Z

SatB.X SatB.Y SatB.Z ISL_Range;

If ISL_Range < THRESHOLD

Report InterplaneISLWindows SatA.UTCGregorian ISL_Range;

EndIf;

EndWhile;

Note: A seed Propagate before the While loop advances both satellites by one timestep and computes the initial ISL_Range. This causes each minute's data to appear twice in the output file (from the seed and from the loop body). The duplicate rows have identical ISL_Range values and are filtered out in post-processing. All property assignment lines use the mandatory GMAT prefix required by GMAT R2022a.