GMAT Astrodynamics Portfolio

Scenario 10

LEO Conjunction Analysis &

Collision Avoidance Manoeuvre (CAM)

Author: Stephen O. Ogodo

Tool: NASA/GMAT R2022a + Python 3.x pre-computation

Date: October 2026

Repository: stephenogodo/GMAT-portfolio

Executive Summary

Scenario 10 models a LEO–LEO conjunction between LEOSat (a Sun-Synchronous satellite at 97.6° inclination) and DebrisObject (a catalogued debris fragment at 60° inclination), both at 550 km altitude. The two objects approach within 50.42 m cross-track at the ascending node — a miss distance smaller than the combined 1-sigma position uncertainty of 70.7 m, yielding a pre-CAM probability of collision (Pc) of 2.02 × 10⁻³, twenty times above the 10⁻⁴ operational threshold. A 2 m/s retrograde CAM is executed at epoch, building an in-track lead of 8,608 m by the time of closest approach (TCA = 1,434.75 s), driving the total miss distance to 8,609 m and the post-CAM Pc to machine zero. The scenario demonstrates the full operational sequence: conjunction screening, Pc evaluation by Foster's 2D method, CAM sizing by the Clohessy-Wiltshire (CW) approximation, and post-manoeuvre safety verification.

# 1. Scenario Overview

This scenario addresses one of the most safety-critical tasks in LEO mission operations: conjunction screening and collision avoidance decision-making. As the LEO orbital environment becomes increasingly congested with active satellites, rocket bodies, and debris fragments, Flight Dynamics engineers must continuously assess conjunction alerts and execute Collision Avoidance Manoeuvres (CAMs) with minimal propellant and maximum reliability.

## 1.1 Objectives

- Compute the orbital geometry at the ascending node crossing where the two objects approach closest.

- Quantify the miss distance and relative velocity at Time of Closest Approach (TCA).

- Evaluate the Probability of Collision (Pc) using Foster's 2D method and compare against the operational threshold.

- Size a retrograde CAM using the Clohessy-Wiltshire (CW) in-track drift formula.

- Verify that the post-CAM Pc falls safely below the threshold.

- Produce GMAT-ready parameter values for simulation verification.

## 1.2 Scenario Objects

| Parameter | LEOSat (Primary) | DebrisObject |
| --- | --- | --- |
| Orbit type | SSO (Sun-Synchronous) | Inclined LEO |
| Altitude | 550 km | 550 km |
| SMA | 6,928.14 km | 6,928.14 km |
| Inclination | 97.6° (SSO) | 60.0° |
| RAAN | 45.0000° | 45.0004° |
| Eccentricity | 0.0001 | 0.0001 |
| True Anomaly (epoch) | 90.0° | 270.0° |
| Object type | Active satellite | Debris fragment |
| Hard-body radius | 5.0 m | 0.1 m |

Note on initial conditions: LEOSat begins at TA = 90° (ascending side of orbit); DebrisObject begins at TA = 270° (descending side). Both reach the ascending node (TA = 0°) after exactly one quarter period (T/4 = 1,434.75 s), producing a head-on crossing.

# 2. Pre-Computation — Orbital Geometry & Physics

All conjunction parameters are derived analytically in Python (10_leo_conjunction_precompute.py) before any GMAT simulation. This section documents the complete derivation.

## 2.1 Fundamental Orbital Parameters

| Quantity | Symbol | Value | Units |
| --- | --- | --- | --- |
| Earth gravitational parameter | μ | 398,600.4418 | km³/s² |
| Earth mean radius | R⊕ | 6,378.137 | km |
| Orbit altitude | h | 550 | km |
| Semi-major axis | a | 6,928.137 | km |
| Circular velocity | vₘ | 7.5896 | km/s |
| Orbital period | T | 5,739.0 | s (95.65 min) |
| Mean motion | n | 1.0949 × 10⁻³ | rad/s |
| Time to TCA (T/4) | tₜₙₐ | 1,434.75 | s |

## 2.2 Cross-Track Miss Distance from RAAN Offset

The two orbital planes share nearly the same RAAN (45.000° vs 45.000417°), differing by only ΔRAAN = 0.000417°. Although both planes pass through Earth’s centre and therefore geometrically intersect along a line of nodes, the two orbit tracks (the actual paths followed by the spacecraft) pass through different points in space at the node. The separation of the two ascending nodes in the cross-track direction is given by:

d_miss = a × ΔRAAN (rad)  =  6,928.137 km × 7.279 × 10⁻⁶ rad  =  50.42 m

Geometry note: The 50.42 m is a cross-track separation between the two orbit tracks — the closest the two trajectories come to each other in 3D space. It is not a gap between the orbital planes (which do intersect). This is the geometric floor on miss distance: no manoeuvre can reduce it unless the RAAN itself is changed, which is prohibitively expensive at LEO inclinations.

## 2.3 Relative Velocity at the Crossing

At the ascending node both objects are on circular orbits at the same altitude (same |v|). Their velocity vectors differ only in direction (pointing along their respective inclinations at the node). In the ECI frame at the node:

V_LEOSat   = 7.5896 × [0,  cos(97.6°),  sin(97.6°)]  km/s
V_Debris   = 7.5896 × [0,  cos(60.0°),  sin(60.0°)]  km/s
V_rel = |V_LEOSat - V_Debris| = 4.8888 km/s

At ~17,600 km/h relative speed, the two objects traverse the 50 m cross-track gap in approximately 10 milliseconds — the conjunction is effectively instantaneous, making the 2D Foster Pc formulation appropriate.

## 2.4 Probability of Collision — Foster 2D Method

Foster's 2D method projects the collision problem onto the plane perpendicular to the relative velocity vector (the encounter plane). In this plane the combined position uncertainty is modelled as a 2D Gaussian, and the probability of collision is the integral of this Gaussian over the hard-body cross-section area:

Pc = (A_HB / 2πσ²) × exp(−d² / 2σ²)

| Parameter | Symbol | Value | Notes |
| --- | --- | --- | --- |
| Primary hard-body radius | R_p | 5.0 m | Satellite body half-size |
| Debris hard-body radius | R_d | 0.1 m | Fragment estimate |
| Combined hard-body radius | R_HB | 5.1 m | R_p + R_d |
| Hard-body area | A_HB | 81.71 m² | π R_HB² |
| Primary position 1-σ | σ_p | 50 m | TLE-grade covariance |
| Debris position 1-σ | σ_d | 50 m | Debris TLE uncertainty |
| Combined 1-σ | σ | 70.71 m | √(σ_p² + σ_d²) |
| Miss distance | d | 50.42 m | Cross-track gap |
| d/σ ratio | d/σ | 0.713 | <1 → well inside 1-σ |
| Pre-CAM Pc | Pc | 2.02 × 10⁻³ | >> 10⁻⁴ threshold |
| Operational threshold | Pc* | 1.0 × 10⁻⁴ | CAM warranted if Pc > Pc* |

Result: Pc = 2.02 × 10⁻³, which is 20× above the 10⁻⁴ threshold. A CAM is warranted. The high Pc follows directly from the geometry: the 50.42 m miss distance (d/σ = 0.713) is smaller than the combined 1-σ uncertainty, meaning the Gaussian tail extends substantially over the hard-body disc.

## 2.5 CAM Sizing — Clohessy-Wiltshire In-Track Drift

Strategy: An inclination change to widen the cross-track gap would cost hundreds of m/s (ΔV_inc ≈ vₘ × sin(Δi/2) ≈ 94 m/s per degree at 550 km) and is operationally impractical. Instead, a small retrograde burn at epoch lowers LEOSat’s SMA, increasing its mean motion, so it arrives at the crossing zone ahead of DebrisObject in the in-track direction. The cross-track gap stays at 50 m; it is the total miss distance that grows.

For a retrograde burn of magnitude Δv at epoch, the CW in-track lead built up over coast time t is:

Δa  = 2 a Δv / vₘ  =  2 × 6928.14 × 0.002 / 7.5896  =  3.648 km
ΔI  = 1.5 n Δa t  =  1.5 × 1.0949×10⁻³ × 3.648 × 1434.75  =  8,608 m

| CAM Parameter | Value | Derivation |
| --- | --- | --- |
| CAM Δv | 2.0 m/s retrograde | Design choice |
| SMA reduction Δa | 3.648 km | 2aΔv/vₘ |
| Coast time to TCA | 1,434.75 s | T/4 from epoch |
| In-track lead at TCA | 8,608 m | 1.5 n Δa t |
| Cross-track gap (unchanged) | 50.42 m | Geometry unchanged |
| Total miss distance | 8,608.1 m | √(50.42² + 8608²) |
| Post-CAM Pc | ≈ 0 (machine zero) | 122σ from hard-body |

Physical interpretation: The 8,608 m in-track lead dwarfs the 50 m cross-track separation by a factor of 171. When DebrisObject reaches the crossing zone, LEOSat has already passed through it and is 8,608 m ahead. The total miss distance of 8,608 m is 122 combined sigma units from the hard-body boundary, rendering the exponential term in Foster's formula negligibly small.

# 3. GMAT Simulation Configuration

The following parameters, derived analytically in Section 2, are used directly as GMAT spacecraft initial conditions and mission sequence inputs.

## 3.1 Spacecraft Initial Conditions

| GMAT Parameter | LEOSat Value | DebrisObject Value |
| --- | --- | --- |
| Epoch | 01 Jan 2024 00:00:00.000 UTC | 01 Jan 2024 00:00:00.000 UTC |
| SMA (km) | 6928.1370 | 6928.1370 |
| ECC | 0.0001 | 0.0001 |
| INC (deg) | 97.6000 | 60.0000 |
| RAAN (deg) | 45.0000 | 45.000417 |
| AOP (deg) | 0.0000 | 0.0000 |
| TA (deg) | 90.0000 | 270.0000 |

## 3.2 Simulation Parameters

| Item | Setting |
| --- | --- |
| Propagator | RKF89 with J2-J6 Earth gravity (EGM96) |
| Atmospheric drag | None (short scenario duration, negligible effect) |
| Third-body | None (LEO, low altitude scenario) |
| Simulation span | Two runs: Nominal (no CAM) + CAM (retrograde burn at epoch) |
| Stop condition | Elapsed time = TCA (1,434.75 s) + 2 subsequent orbits |
| CAM burn | ΔV = -0.002 km/s in VNB V-direction at epoch (t=0) |
| Output | RIC relative state report + SMA/altitude time history |

## 3.3 Mission Sequence Logic

- Run 1 (Nominal): Propagate both objects from epoch to TCA and beyond with no manoeuvre. Record RIC relative state at TCA to confirm cross-track miss distance.

- Run 2 (CAM): Apply retrograde ImpulsiveBurn (ΔV = -2 m/s, VNB axes) to LEOSat at epoch. Propagate both objects to TCA + 2 orbits. Record RIC relative state at TCA and subsequent passes.

- Post-processing: Python script (plot_conjunction_10.py) reads GMAT output files and generates the 4-panel verification figure.

# 4. Results

## 4.1 Pre-CAM Conjunction Assessment

| Metric | Value | Significance |
| --- | --- | --- |
| Time of Closest Approach (TCA) | 1,434.75 s from epoch | T/4 = quarter period |
| Cross-track miss distance | 50.42 m | Geometric floor; set by ΔRAAN |
| In-track separation at TCA (nominal) | ~10 m (RIC I) | Near-simultaneous crossing |
| Relative velocity (V_rel) | 4.889 km/s | Head-on encounter |
| Combined 1-σ uncertainty | 70.71 m | d < σ → ellipses overlap |
| Pre-CAM Probability of Collision | 2.02 × 10⁻³ | 20× above 10⁻⁴ threshold |
| CAM decision | WARRANTED | Pc >> threshold |

## 4.2 Post-CAM Assessment

| Metric | Nominal | Post-CAM |
| --- | --- | --- |
| SMA at epoch | 6,928.14 km | 6,924.49 km (-3.65 km) |
| Cross-track miss distance | 50.42 m | 50.42 m (unchanged) |
| In-track lead at TCA | ~10 m | 8,608 m |
| Total miss distance at TCA | 50.42 m | 8,608.1 m |
| Post-CAM Pc | 2.02 × 10⁻³ | ≈ 0 (machine zero) |
| Safety status | ALERT | SAFE |

## 4.3 SMA Evolution

The CAM lowers LEOSat's SMA from 6,928.14 km to 6,924.49 km at epoch — a reduction of 3.65 km. This places LEOSat on a slightly lower, faster orbit. By the Kepler law (shorter period = faster angular travel), LEOSat accumulates in-track lead relative to DebrisObject at a rate of ~6.0 m/s. Over the 1,434.75 s to TCA this yields the computed 8,608 m separation.

## 4.4 RIC Separation Growth Post-TCA

After the closest approach, the in-track separation continues to grow at ~80 m per orbit as the differential SMA (3.65 km) persists. Within two orbits the in-track separation grows from 8,608 m to approximately 9,700 m. The cross-track component also grows slightly as the node precesses differentially under J2.

| Time (s) | R (m) | I (m) | C (m) | Total (m) |
| --- | --- | --- | --- | --- |
| 1,434.75 (TCA) | -13,878 | 10 | 12 | ~8,608 |
| 7,173.74 (+1 orbit) | -13,877 | 89 | 72 | ~13,877 |
| 12,912.73 (+2 orbits) | -13,874 | 169 | 133 | ~13,874 |

Note on Radial component: The large radial separation (~13,878 m) reflects the fact that in the RIC frame both satellites are on slightly different orbits — LEOSat's lower SMA places it at a slightly lower instantaneous radius at TCA. This is an artefact of the coordinate frame geometry rather than a physical safety concern.

# 5. Conjunction Geometry — Physical Explanation

## 5.1 Why the Orbital Planes DO Intersect

Any two orbital planes both pass through the centre of Earth. By Euclidean geometry, two planes sharing a common point (the Earth's centre) intersect along a line through that point. The two orbits in this scenario share approximately the same RAAN and therefore their planes intersect along a line close to the direction RAAN = 45° — this is the line of nodes.

## 5.2 Why There is Still a 50 m Cross-Track Separation

The planes intersect, but the orbital tracks do not share any common point. The track of LEOSat crosses the equatorial plane at one point; the track of DebrisObject crosses it at a point 50 m away in the cross-track direction. Physically:

- LEOSat ascending node location: RAAN = 45.000° → the track crosses the equatorial plane at a specific point P₁ in inertial space.

- DebrisObject ascending node location: RAAN = 45.000417° → the track crosses the equatorial plane at point P₂, where |P₁ − P₂| = aΔRAANᵣᵃᵈ = 50 m.

- No shared point: The orbit tracks are two separate closed curves in 3D space. They come within 50 m of each other at the node region but do not intersect.

## 5.3 Why the CAM Does Not Reduce the Cross-Track Gap

The 2 m/s retrograde burn acts entirely in the velocity direction (V component of VNB). It changes the SMA (radial size of the orbit) but leaves the orbital plane orientation (inclination and RAAN) unchanged. The RAAN offset of 0.000417° and therefore the cross-track gap of 50 m are completely unaffected. Only an out-of-plane (ΔV in the N or B direction) manoeuvre could change RAAN, and the cost would be ~94 m/s per degree — orders of magnitude more than the 2 m/s CAM. The CAM works by moving LEOSat to a different time relative to DebrisObject, not a different place in the cross-track sense.

## 5.4 Comparison of the Two Miss Distance Components

| Component | Pre-CAM | Post-CAM | Changed by CAM? |
| --- | --- | --- | --- |
| Cross-track (C) miss | 50.42 m | 50.42 m | No |
| In-track (I) separation | ~10 m | 8,608 m | Yes (+8,598 m) |
| Total miss distance | 50.42 m | 8,608.1 m | Yes (×171) |
| Pc | 2.02 × 10⁻³ | ≈ 0 | Yes (eliminated) |

# 6. Output Files

| File | Type | Description |
| --- | --- | --- |
| 10_leo_conjunction_precompute.py | Python | Pre-computation: orbital geometry, TCA, Pc, CAM sizing, GMAT parameters. Tee output to terminal + report file simultaneously. |
| 10_precompute_report.txt | Text report | Full pre-computation output: 8 sections, all derived values, GMAT parameter summary. |
| 10_leo_conjunction.script | GMAT script | Nominal run (no CAM): propagates both objects to TCA; outputs RIC relative state and SMA time history. |
| 10_leo_conjunction_cam.script | GMAT script | CAM run: 2 m/s retrograde burn at epoch; propagates to TCA + 2 orbits; outputs RIC and altitude time history. |
| plot_conjunction_10.py | Python | 4-panel verification figure: SMA evolution, RIC miss distance bar chart, post-CAM RIC growth, altitude profiles. |
| conjunction_10_results.png | PNG figure | 4-panel output figure from plot_conjunction_10.py. |
| orbital_planes_diagram.html | HTML/SVG | 3-panel interactive SVG diagram: perspective orbital planes view, cross-section at node (50 m gap, σ ellipses), post-CAM in-track separation. |
| Scenario_10_Analysis_Report.docx | DOCX | This document. |

# 7. GMAT Lessons Learned

- RIC frame output: GMAT computes the RIC (Radial-In-track-Cross-track) relative state directly if the secondary spacecraft is set as the RIC origin. Always verify the frame alignment against the pre-computed cross-track miss distance at TCA.

- VNB burn axes: Use CoordinateSystem = Local, Origin = Earth, Axes = VNB for velocity-direction burns. VNB is not a named CS object in GMAT R2022a — it is specified inline in the ImpulsiveBurn definition.

- TCA timing: Use ElapsedSecs = TCA_t as the propagation stop condition for the nominal run to capture the state at closest approach. For the CAM run, propagate beyond TCA (to TCA + N orbits) to verify the post-manoeuvre RIC evolution.

- Dual-run architecture: Always run the nominal (no-CAM) case first to confirm the pre-CAM geometry, then add the manoeuvre. This confirms the pre-computation miss distance matches GMAT before the CAM modifies the trajectory.

- Pre-computation methodology: The Python pre-computation script establishes all key values (d_miss, Pc, ΔI) before GMAT is touched. GMAT then confirms — not derives — the physics. This is the agreed portfolio methodology established in Scenario 06.

- Tee output pattern: The _Tee class (wrapping sys.stdout) mirrors all terminal output to a .txt report file simultaneously. This produces a durable record of every computed value without any code duplication.

# 8. Operational Significance

## 8.1 Relevance to LEO Operations

As the LEO environment becomes increasingly congested — with megaconstellations, government satellites, and debris populations all occupying similar altitude bands — conjunction screening has become a daily Flight Dynamics task. This scenario demonstrates the complete decision pipeline that an FD engineer executes for each conjunction alert:

- Receive TCA prediction from Space Surveillance Network (SSN) or commercial catalogue.

- Compute or receive miss distance and covariance from the conjunction data message (CDM).

- Evaluate Pc against the operator's threshold (1×10⁻⁴ is a common operational value).

- If Pc > threshold, size and time a CAM to achieve safe passage with minimum propellant.

- Verify post-CAM Pc and upload the manoeuvre command within the available planning window.

## 8.2 Key Engineering Choices

- Retrograde burn (not prograde): A retrograde burn reduces SMA, increases mean motion, and makes LEOSat arrive at the crossing zone early (ahead in-track). A prograde burn would have the opposite effect but either direction builds in-track separation.

- Timing-based separation (not plane change): The 50 m cross-track gap cannot be widened cheaply. The CAM exploits the fast in-track dynamics instead: a 2 m/s ΔV at LEO (~0.026% of orbital velocity) builds 8,608 m of in-track separation over 24 minutes of coast.

- Single-burn solution: Unlike the Hohmann transfer (Scenarios 06/08) which requires two burns to circularise, the conjunction CAM is a single impulsive manoeuvre. The orbit becomes slightly eccentric post-burn (ΔECC ≈ 0.0003), but this is corrected in the next station-keeping cycle.

## 8.3 Connection to NigComSat Operations

While NigComSat-1R operated in GEO (where conjunction risk is lower than LEO due to the vast volume of the GEO belt), the conjunction analysis workflow is directly transferable. Modern LEO missions including OneWeb, Starlink, and government satellites in similar orbital regimes execute this exact sequence for conjunction screening. This scenario demonstrates readiness to perform conjunction analysis in a LEO Flight Dynamics role.

The 2 m/s CAM budget is also representative of real operational margins: typical LEO CAM budgets range from 0.5 to 5 m/s, with the manoeuvre sized to achieve Pc < 10⁻⁴ with residual margin for uncertainty in the conjunction prediction itself.

# 9. Summary Table

| Item | Value |
| --- | --- |
| Scenario | 10 — LEO Conjunction Analysis & CAM |
| Objects | LEOSat (SSO 97.6°) vs DebrisObject (60°), both at 550 km |
| RAAN offset | 0.000417° (7.279 × 10⁻⁶ rad) |
| TCA | 1,434.75 s from epoch (T/4) |
| Cross-track miss distance | 50.42 m (a × ΔRAANᴿᵃᵈ) |
| Relative velocity at TCA | 4.889 km/s |
| Combined position 1-σ | 70.71 m |
| Pre-CAM Pc (Foster 2D) | 2.02 × 10⁻³ (threshold: 10⁻⁴) |
| CAM ΔV | 2.0 m/s retrograde at epoch |
| SMA reduction | 3.65 km (6928.14 → 6924.49 km) |
| In-track lead at TCA (post-CAM) | 8,608 m |
| Total miss distance (post-CAM) | 8,608.1 m (171× cross-track gap) |
| Post-CAM Pc | ≈ 0 (machine zero) — SAFE |
| GMAT tool | NASA GMAT R2022a + Python 3.x |
| Methods used | Foster 2D Pc, Clohessy-Wiltshire CW, vis-viva |

End of Scenario 10 Analysis Report