GMAT Portfolio — Scenario 06

Hohmann Transfer: Delta-V Derivation

Stephen O. Ogodo  |  Astrodynamics & Flight Dynamics Portfolio

October 2026

## 1.  Objective

Restore a LEO satellite from a degraded orbit (SMA = 6758.14 km) to a nominal orbit (SMA = 6778.14 km) using a two-impulse Hohmann transfer. This document derives the required delta-V (ΔV) for each burn from first principles.

## 2.  The Vis-Viva Equation

All delta-V calculations in Keplerian mechanics rest on the vis-viva equation, which gives the orbital speed at any point in an elliptical orbit:

v = √[ μ (2/r  −  1/a) ]

Where:

μ = GM = 398,600.4418 km³/s²   (Earth's gravitational parameter)

r = current distance from Earth's centre (km)

a = semi-major axis of the orbit (km)

For a circular orbit, r = a throughout, so the equation simplifies to:

v_circular = √(μ / r)

## 3.  Orbit Parameters

Three orbits are involved: the initial circular orbit, the intermediate transfer ellipse, and the final circular orbit.

| Orbit | SMA a (km) | Perigee radius r_p (km) | Apogee radius r_a (km) |
| --- | --- | --- | --- |
| Initial circular (degraded) | 6758.14 | 6758.14 | 6758.14 |
| Transfer ellipse | 6768.14 | 6758.14 | 6778.14 |
| Final circular (nominal) | 6778.14 | 6778.14 | 6778.14 |

The transfer ellipse SMA is the mean of the two circular radii:

a_transfer = (r₁ + r₂) / 2 = (6758.14 + 6778.14) / 2 = 6768.14 km

## 4.  Computing the Four Key Velocities

4.1   v₁ — Speed in the initial circular orbit  (r₁ = 6758.14 km)

v₁ = √(μ / r₁) = √(398600.4418 / 6758.14) = 7.6762 km/s

4.2   v_tp — Speed of the transfer ellipse at perigee  (r = r₁, a = a_transfer)

v_tp = √[ μ (2/r₁ − 1/a_transfer) ]

v_tp = √[ 398600.4418 × (2/6758.14 − 1/6768.14) ] = 7.6819 km/s

4.3   v_ta — Speed of the transfer ellipse at apogee  (r = r₂, a = a_transfer)

v_ta = √[ μ (2/r₂ − 1/a_transfer) ]

v_ta = √[ 398600.4418 × (2/6778.14 − 1/6768.14) ] = 7.6542 km/s

Note: the satellite slows down as it coasts from perigee to apogee — kinetic energy converts to potential energy (conservation of energy).

4.4   v₂ — Speed in the final circular orbit  (r₂ = 6778.14 km)

v₂ = √(μ / r₂) = √(398600.4418 / 6778.14) = 7.6652 km/s

## 5.  Delta-V Calculations

5.1   ΔV₁ — First burn: prograde impulse at perigee

Burn 1 accelerates the satellite from the circular orbit speed onto the transfer ellipse. It fires at perigee where r = r₁:

ΔV₁ = v_tp − v₁ = 7.6819 − 7.6762 = 0.005671 km/s  (5.671 m/s)

The position does not change instantaneously. Only the velocity jumps. The orbit is now an ellipse with perigee at 6758.14 km and apogee at 6778.14 km.

5.2   ΔV₂ — Second burn: prograde impulse at apogee

Burn 2 circularises the orbit. The satellite arrives at apogee moving at v_ta = 7.6542 km/s, but needs v₂ = 7.6652 km/s to maintain a circular orbit at r₂:

ΔV₂ = v₂ − v_ta = 7.6652 − 7.6542 = 0.005667 km/s  (5.667 m/s)

This burn raises the perigee from 6758.14 km to 6778.14 km, matching the apogee and completing the circularisation.

## 6.  Results Summary

| Burn | Location | Direction | ΔV (km/s) | ΔV (m/s) | Effect |
| --- | --- | --- | --- | --- | --- |
| Burn 1 | Perigee (r₁ = 6758.14 km) | Prograde (+V) | 0.005671 | 5.671 | Raises apogee to 6778.14 km |
| Burn 2 | Apogee (r₂ = 6778.14 km) | Prograde (+V) | 0.005667 | 5.667 | Raises perigee to 6778.14 km |
| Total | — | — | 0.011338 | 11.338 | Orbit restored to nominal |

## 7.  Why Burn Timing Is Critical

The vis-viva equation computes velocity as a function of both r and a. A burn changes a (by adding energy) but not r (position is unchanged at the instant of firing).

Burn 1 must fire exactly at perigee because the ΔV value was derived assuming r = r₁ at that moment. Firing even slightly off-periapsis means r is larger than r₁ in the equation, the resulting transfer ellipse has a shifted apogee, and the target altitude is missed.

Burn 2 must fire exactly at apogee because that is the one point in the transfer orbit where r = r₂. A burn there raises only the perigee (the opposite side of the orbit). Firing early or late produces an ellipse that does not circularise at the correct altitude.

In GMAT, this precision is enforced by the stopping conditions {LEOSat.Periapsis} and {LEOSat.Apoapsis}, which halt propagation at the exact geometric extrema of the orbit before each burn command is executed.

## 8.  Why ΔV₁ ≠ ΔV₂

The two burns differ by 0.004 m/s. The asymmetry arises because orbital velocity scales as 1/√r — the vis-viva curve is nonlinear. The energy cost of raising the orbit by 10 km at r₁ is not the same as the cost of raising it by 10 km at r₂.

For this 20 km manoeuvre the difference is negligible. For a large manoeuvre the asymmetry is dramatic: a Hohmann transfer from LEO (r ≈ 6778 km) to GEO (r ≈ 42,164 km) requires ΔV₁ ≈ 2.4 km/s and ΔV₂ ≈ 1.5 km/s — nobody would guess those are "roughly equal" without doing the maths.

GMAT Astrodynamics Portfolio  ·  Stephen O. Ogodo  ·  2026