GMAT ASTRODYNAMICS PORTFOLIO

SCENARIO 06

Hohmann Transfer Orbit Restoration

Two-Impulse Manoeuvre | Impulsive Burn Model | GMAT R2022a

Stephen Ogodo

Flight Dynamics Engineer | LEO Satellite Operations

October 2026

PORTFOLIO DOCUMENT — FOR PROFESSIONAL REVIEW PURPOSES

1. Executive Summary

This report documents Scenario 06 of the GMAT Astrodynamics Portfolio: a two-impulse Hohmann transfer manoeuvre that restores a LEO satellite from a drag-degraded orbit to its nominal operational altitude. The scenario demonstrates the complete manoeuvre planning pipeline — analytical Delta-V derivation in Python, GMAT script implementation, and numerical verification — mirroring the process used in real LEO satellite flight dynamics operations.

| KEY RESULTS |  |
| --- | --- |
| SMA correction | 6758.14 km --> 6778.14 km (+20.00 km) |
| Target SMA achieved | 6778.060 km (error: -80 m, 0.001%) |
| Delta-V (Burn 1 + Burn 2) | 5.671 m/s + 5.667 m/s = 11.338 m/s total |
| Residual ECC post-manoeuvre | 0.000077 (near-circular; < 0.0001 threshold) |
| Manoeuvre duration | Burn 1 at 02:18:45 UTC ¦ Burn 2 at 03:04:28 UTC |
| INC change | 51.600 deg --> 51.601 deg (J2 nodal regression; acceptable) |
| Simulation tool | GMAT R2022a ¦ RK89 ¦ 4x4 Earth gravity |
| Pre-computation method | Python vis-viva (Hohmann_Transfer_Delta-V_Calculator.py) |

2. Scenario Background and Mission Context

2.1  Operational Motivation

In real LEO satellite operations, atmospheric drag continuously removes energy from the orbit, causing the semi-major axis (SMA) to decay over time. If station-keeping burns are delayed — due to an anomaly, ground contact outage, or planned gap — the satellite sinks below its nominal altitude. This scenario models the corrective response: a Hohmann transfer that restores the satellite from a degraded orbit (SMA = 6758.14 km, approximately 387 km altitude) back to the nominal operational orbit (SMA = 6778.14 km, approximately 407 km altitude).

This 20 km SMA correction with only 11.338 m/s of total Delta-V illustrates the fuel efficiency of the two-impulse Hohmann transfer vs. a direct single burn. For comparison, the same altitude change via a single impulsive burn would require slightly more Delta-V and leave the orbit with non-trivial eccentricity — both are operationally undesirable.

2.2  Parallel to NigComSat Operations

The Hohmann transfer principle underlies GEO East-West station-keeping (EWSK) as well as LEO altitude restoration. At NigComSat-1R, every station-keeping Telecommand upload specifies a finite-burn duration computed from the impulsive Delta-V derived by exactly this method. The impulsive model (this scenario) is the starting point; the finite-burn simulation with a differential corrector (Scenario 08) is the next step in the pipeline. Together they demonstrate the complete manoeuvre planning workflow used in real operations.

2.3  Relationship to Scenario 08 (Finite Burn)

Impulsive burns (Scenario 06) are a mathematical idealisation — the burn duration is assumed zero and all Delta-V is applied instantaneously at the optimal point. This yields exact closed-form solutions from the vis-viva equation and no differential corrector (DC) is needed.

Finite burns (Scenario 08) are what physically occurs. A thruster fires for minutes, the orbit changes continuously during the burn arc, residual eccentricity is unavoidable, and a DC must iterate to find the correct burn duration. The Scenario 06 Delta-V is the input to that process, not the final answer.

This is the reason Scenario 06 required no iteration and converged on the first GMAT run: the physics were fully determined analytically before any simulation began. The physics-first methodology — Python pre-computation, then GMAT implementation — is a core principle of this portfolio.

3. Orbital Mechanics and Delta-V Derivation

3.1  Hohmann Transfer Geometry

A Hohmann transfer between two circular orbits is the most fuel-efficient two-impulse manoeuvre when the altitude difference is less than approximately 11.94 times the lower orbit radius (always true for LEO corrections). The transfer orbit is an ellipse whose perigee coincides with the initial circular orbit and whose apogee coincides with the target circular orbit.

For Scenario 06: perigee radius r1 = 6758.14 km, apogee radius r2 = 6778.14 km, transfer ellipse semi-major axis a_t = (r1 + r2)/2 = 6768.14 km.

3.2  Vis-Viva Derivation (from Hohmann_Transfer_Delta-V_Calculator.py)

The vis-viva equation gives orbital speed at any point in an orbit:

v = sqrt( mu * (2/r - 1/a) )

where mu = 398600.4418 km3/s2 (Earth gravitational parameter). Applying this equation at the four key orbital points:

| Orbital Point | Radius r (km) | SMA a (km) | Speed v (km/s) |
| --- | --- | --- | --- |
| Circular at r1 (degraded orbit) | 6758.14 | 6758.14 | 7.678966 |
| Transfer ellipse at perigee (r1) | 6758.14 | 6768.14 | 7.684637 |
| Transfer ellipse at apogee (r2) | 6778.14 | 6768.14 | 7.661471 |
| Circular at r2 (nominal orbit) | 6778.14 | 6778.14 | 7.667133 |

3.3  Delta-V Values

Burn 1 (prograde at perigee, raises apogee):  DV1 = v_transfer_perigee - v_circular(r1) = 7.684637 - 7.678966 = 0.005671 km/s (5.671 m/s)

Burn 2 (prograde at apogee, raises perigee):  DV2 = v_circular(r2) - v_transfer_apogee = 7.667133 - 7.661471 = 0.005662 km/s (5.662 m/s)

Total Delta-V: DV1 + DV2 = 11.333 m/s

3.4  Why DV1 is Not Equal to DV2

At first inspection the two burns appear symmetric: both change the orbit by 20 km. However, orbital velocity follows v = sqrt(mu/r) on a circular orbit — a 1/sqrt(r) relationship. This curve is concave: the velocity-radius gradient is steeper at lower altitude than at higher altitude.

At r1 = 6758.14 km (lower, steeper part of the curve), the velocity difference between the circular orbit and the transfer ellipse is slightly larger than at r2 = 6778.14 km (higher, shallower part). Hence DV1 > DV2. The difference is small (9 m/s) because the 20 km altitude change is tiny relative to the orbit radius (~6768 km). For large transfers (LEO to GEO) the asymmetry is pronounced — DV1 is far smaller than DV2.

3.5  Half-Period Coast Time

Between Burn 1 (at perigee) and Burn 2 (at apogee), the satellite coasts through exactly one half-period of the transfer ellipse:

T_transfer = 2*pi * sqrt(a_t^3 / mu) = 2*pi * sqrt(6768.14^3 / 398600.4) = 5552 s

Half-period (Burn 1 to Burn 2): 2776 s (46.3 minutes). The GMAT simulation shows: Burn 1 at 02:18:45 UTC, Burn 2 at 03:04:28 UTC — difference = 2743 s (45.7 min), consistent with the analytical estimate (small deviation due to J2 perturbation during coast).

4. GMAT Simulation Configuration

4.1  Spacecraft Parameters

| Parameter | Value | Notes |
| --- | --- | --- |
| Spacecraft name | LEOSat | Single satellite, impulsive burns |
| Epoch | 01 Jan 2024 00:00:00.000 UTC |  |
| SMA (initial) | 6758.14 km | Degraded orbit (~387 km alt) |
| ECC | 0.0001 | Near-circular (not exactly zero — small initial eccentricity) |
| INC | 51.6 deg | ISS- altitude inclination; consistent with all scenarios |
| RAAN | 0 deg |  |
| AOP | 0 deg | Periapsis at equatorial crossing |
| TA | 0 deg | At periapsis at epoch |
| Dry Mass | 500 kg |  |
| Cd | 2.2 | Drag coefficient |
| Drag Area | 5 m2 |  |

4.2  Force Model

| Force Component | Setting |
| --- | --- |
| Central body | Earth |
| Gravity model | EGM-96, Degree 4, Order 4 |
| Propagator | RungeKutta89 (RK89) |
| Atmospheric drag | Cd = 2.2, Area = 5 m2 (included via spacecraft properties) |
| Solar radiation pressure | Cr = 1.8, SRP Area = 5 m2 |
| Third-body | Not modelled (acceptable for short-duration LEO manoeuvre) |

4.3  Impulsive Burn Definitions

Both burns are defined as ImpulsiveBurn objects in the VNB (Velocity-Normal-Binormal) reference frame. VNB is the standard frame for manoeuvre specification in LEO: the V-axis is aligned with the velocity vector, N is the orbit normal, and B completes the right-handed triad.

Critical GMAT syntax: VNB is not a named coordinate system object in GMAT R2022a. The correct specification is CoordinateSystem = Local; Origin = Earth; Axes = VNB inline in the ImpulsiveBurn definition. No Create CoordinateSystem VNB command is needed or valid.

| Burn | CoordinateSystem | Origin | Axes | Element1 (V) | Element2 (N) | Element3 (B) |
| --- | --- | --- | --- | --- | --- | --- |
| Burn1 (at periapsis) | Local | Earth | VNB | 0.005671 km/s | 0 | 0 |
| Burn2 (at apoapsis) | Local | Earth | VNB | 0.005667 km/s | 0 | 0 |

4.4  Mission Sequence

The mission sequence uses periapsis and apoapsis stopping conditions to time the burns precisely:

1.  Coast 5440 s (coast degraded orbit — ~1 orbit pass)

2.  Propagate {LEOSat.Periapsis}

3.  Report (pre-Burn1 state)

4.  Maneuver Burn1(LEOSat)

5.  Report (post-Burn1 state)

6.  Propagate {LEOSat.Apoapsis}

7.  Report (pre-Burn2 state)

8.  Maneuver Burn2(LEOSat)

9.  Report (post-Burn2 state)

10. Coast 5440 s (verify restored orbit)

11. Report (final state)

The {LEOSat.Periapsis} stopping condition propagates to the exact moment of periapsis passage — the geometrically optimal point for Burn 1 (maximises the Oberth effect on the apogee-raising burn). Similarly, {LEOSat.Apoapsis} times Burn 2 exactly at apogee, where a prograde burn most efficiently raises the periapsis.

5. Simulation Results — ManoeuvreReport.txt

5.1  Milestone State Vector Table

GMAT's Report commands captured the orbital state at six key milestones:

| Milestone | UTC Epoch | SMA (km) | ECC | INC (deg) | TA (deg) |
| --- | --- | --- | --- | --- | --- |
| t=0 Epoch (degraded orbit) | 01 Jan 2024 00:00:00 | 6758.140 | 0.000100 | 51.600 | 0.000 |
| Pre-Burn1 (periapsis) | 01 Jan 2024 02:18:45 | 6758.072 | 0.001609 | 51.600 | 0.000 |
| Post-Burn1 (transfer ellipse) | 01 Jan 2024 02:18:45 | 6768.087 | 0.003086 | 51.600 | ~0.000 |
| Pre-Burn2 (apoapsis) | 01 Jan 2024 03:04:28 | 6768.060 | 0.001401 | 51.601 | 180.000 |
| Post-Burn2 (restored orbit) | 01 Jan 2024 03:04:28 | 6778.060 | 0.000077 | 51.601 | ~0.000 |
| Post-coast (+1.5 h nominal) | 01 Jan 2024 04:35:08 | 6778.049 | 0.000191 | 51.601 | 62.567 |

5.2  Phase-by-Phase Analysis

Phase 1 — Degraded Orbit (Epoch to Pre-Burn1)

Over the 8925-second coast from epoch (TA=0, periapsis) to the Burn1 trigger, the satellite completed approximately 1.5 orbital periods in the degraded orbit. The SMA decreases slightly (6758.140 -> 6758.072 km, -68 m) due to J2 perturbation; ECC rises from 0.000100 to 0.001609 as the orbit is subtly reshaped by J2. These are small but physically correct: J2 causes differential precession and small but measurable oscillations in Keplerian elements.

Phase 2 — Burn 1 (Prograde at Periapsis)

At 02:18:45 UTC, the prograde impulse (DV1 = 5.671 m/s) fires. The SMA jumps from 6758.072 km to 6768.087 km — a gain of 10.015 km, matching the expected half-step (the transfer ellipse SMA is midway between r1 and r2). ECC rises from 0.001609 to 0.003086 because the burn has elongated the orbit: perigee remains at ~r1 while apogee has been raised toward r2, creating the characteristic Hohmann transfer ellipse.

Phase 3 — Transfer Ellipse Coast (Burn1 to Burn2)

During the 2743-second (~45.7-minute) half-orbit coast from periapsis to apogee, the SMA decreases very slightly (6768.087 -> 6768.060 km, -27 m) due to J2. ECC reduces from 0.003086 to 0.001401 — J2 smooths the eccentricity during the coast. Both effects are physically expected and operationally negligible. The TA advances from ~0 to 180 deg (apoapsis), confirming correct half-orbit propagation.

Phase 4 — Burn 2 (Prograde at Apoapsis)

At 03:04:28 UTC, the circularisation burn (DV2 = 5.667 m/s) fires. The SMA jumps from 6768.060 km to 6778.060 km, achieving the target of 6778.14 km with an error of only -80 m (0.001%). ECC drops dramatically from 0.001401 to 0.000077 — the orbit is restored to near-circular. This near-zero residual ECC is the expected outcome of the impulsive model: without finite burn arc effects, the circularisation is essentially perfect.

Phase 5 — Restored Orbit Verification (+1.5 hours coast)

After Burn 2, the satellite coasts for 5440 s (~90 min, approximately one orbital period). The final report shows SMA = 6778.049 km (a further -11 m from J2 secular effect over 90 min) and ECC = 0.000191 (slight oscillation from J2 and residual eccentricity). Both are operationally acceptable: the altitude oscillation from ECC = 0.000191 is only

Altitude oscillation = ECC x SMA = 0.000191 x 6778.049 = +/-1.3 km   (negligible for LEO ops)

5.3  Accuracy Assessment

| Metric | Target | Achieved | Error | Assessment |
| --- | --- | --- | --- | --- |
| Post-Burn2 SMA | 6778.14 km | 6778.060 km | -80 m (-0.001%) | Excellent |
| Post-Burn2 ECC | ~0 | 0.000077 | - | Excellent (near-circular) |
| INC change | 0 (no change) | 51.600->51.601 deg | +0.001 deg | J2 nodal regression; acceptable |
| Total Delta-V | 11.338 m/s | 11.338 m/s (input) | 0 | Exact (impulsive model) |

6. GMAT Visualisations

The following visualizations were generated by GMAT R2022a during the simulation run. They confirm the orbital evolution through each phase of the manoeuvre.

6.1  ManoeuvreView (3D OrbitView)

The 3D OrbitView (ManoeuvreView) displays three distinct orbital arcs in EarthMJ2000Eq frame: the initial degraded circular orbit (SMA = 6758.14 km), the transfer ellipse (SMA = 6768.14 km, clearly elongated relative to the circular arcs), and the restored circular orbit (SMA = 6778.14 km). All three arcs are visible simultaneously because GMAT accumulates the orbit trace throughout the simulation. The transfer ellipse appears as a slightly oval arc between the two circular arcs — the characteristic visual signature of a Hohmann transfer.

6.2  GroundTrackPlot

The 2D GroundTrackPlot (Mercator projection) shows the sinusoidal ground track at INC = 51.6 deg throughout the simulation. The small altitude change (20 km) produces no visible change in the ground track pattern — inclination, not altitude, determines the latitude coverage band. The track remains bounded at +/-51.6 deg latitude throughout.

6.3  SMAPlot (XYPlot: SMA vs Elapsed Seconds)

The SMAPlot shows the characteristic three-level step function of a Hohmann transfer:

Level 1 (0 to ~8925 s): SMA ~6758 km (degraded orbit coast)

Step 1 (at ~8925 s): SMA rises to ~6768 km (Burn 1 raises apogee)

Level 2 (~8925 to ~11668 s): SMA ~6768 km (transfer ellipse coast)

Step 2 (at ~11668 s): SMA rises to ~6778 km (Burn 2 circularises at r2)

Level 3 (~11668 s onward): SMA ~6778 km (restored orbit coast — flat, confirming circularity)

The clear two-step SMA profile is the definitive visual confirmation that both burns fired at the correct orbital geometry and achieved the target altitude.

6.4  ECCPlot (XYPlot: ECC vs Elapsed Seconds)

The ECCPlot shows the eccentricity evolution:

~0.0001 initially (small initial eccentricity; orbit is near-circular)

Spike to ~0.003 after Burn 1 (transfer ellipse has perigee at r1, apogee at r2)

Drops to ~0.000077 after Burn 2 (circularisation essentially complete)

Oscillates ~0.0002 in restored orbit (J2 perturbation; operationally negligible)

The spike-and-recover pattern in ECC is the textbook signature of a successful two-impulse Hohmann transfer. Its near-complete suppression after Burn 2 confirms that the impulsive model achieves near-perfect circularisation.

7. Script Development — Errors Encountered and Corrections Applied

Four errors were encountered during script development. Each was diagnosed and corrected before the successful run. These are documented here because they represent common GMAT R2022a pitfalls for engineers transitioning from finite-burn scripting to impulsive-burn scenarios.

| Error | Cause | Fix |
| --- | --- | --- |
| Nonexistent Hardware "{}" in Spacecraft "LEOSat" | Empty hardware braces {} left in spacecraft definition (copied from a finite-burn template that requires Tank/Thruster objects) | Remove hardware braces entirely. ImpulsiveBurn does not need a Tank or Thruster object — no hardware declaration needed. |
| Nonexistent CoordinateSystem "VNB" in ImpulsiveBurn "Burn1" | VNB was written as if it were a named coordinate system ("Create CoordinateSystem VNB"). VNB is not a standalone named CS in GMAT R2022a. | Use inline syntax: CoordinateSystem = Local; Origin = Earth; Axes = VNB. No separate Create CoordinateSystem command. |
| Same VNB error for ImpulsiveBurn "Burn2" | Same root cause as above — both burn definitions copied the incorrect CS syntax. | Same fix applied to Burn2. |
| "OrbitColor" field on ManoeuvreView is no longer in use | Deprecation warning: OrbitColor field was removed in a GMAT version after the script was written. | Warning only — no effect on mission outcome. Remove field to suppress warning in future runs. |
| Non-ASCII character in script comment (GMAT Interpreter Exception) | An em-dash (0x97 Windows-1252) was introduced into a script comment. GMAT requires strict 7-bit ASCII in all script files. | Open file in binary mode, replace 0x97 with ASCII hyphen 0x2D. Establish practice: always write script comments in plain ASCII. |

After correcting errors 1-3, the script interpreted successfully. The non-ASCII character issue (error 5) is a recurring pitfall across all GMAT scenarios in this portfolio (also encountered in Scenarios 08 and 09) — a reminder that any copy-paste from a word processor or rich-text editor into GMAT requires explicit ASCII character verification.

8. Physics-First Methodology

8.1  Pre-Computation Workflow

This was the first scenario in the portfolio to apply the agreed physics-first methodology rigorously: derive all burn parameters analytically before opening GMAT. The Python script Hohmann_Transfer_Delta-V_Calculator.py performs the full vis-viva computation and outputs the exact GMAT syntax ready to copy:

GMAT Burn1.Element1 = 0.005671;

    GMAT Burn2.Element1 = 0.005667;

Values from the Python script are pasted directly into the GMAT ImpulsiveBurn definitions — no manual entry, no rounding, no approximation. The result is that GMAT confirms the numbers on the first run with no differential corrector iteration required.

8.2  Why This Matters Operationally

In real flight dynamics operations, burn parameters are derived analytically first, then verified numerically. An FD engineer who relies on GMAT's differential corrector as the primary computation tool — rather than as a verification step — risks converging on a wrong local minimum or accepting a numerically acceptable but physically incorrect solution. The impulsive model, when correctly applied, gives the exact answer; the finite-burn DC then confirms it with the realism of thruster physics.

The 80 m SMA error (target 6778.14 km, achieved 6778.060 km) in this scenario is due entirely to J2 perturbation during the transfer coast — not to any imprecision in the Delta-V derivation. This residual is inherent to the perturbation environment and is operationally acceptable for a single-cycle manoeuvre. In real operations, the next station-keeping cycle would correct for any residual.

8.3  Connection to the Full Planning Pipeline

| Step | Method | Scenario | Purpose |
| --- | --- | --- | --- |
| 1. Derive Delta-V | Python vis-viva (analytical) | Scenario 06 pre-computation | Exact closed-form solution; no iteration |
| 2. Verify impulsive | GMAT ImpulsiveBurn | Scenario 06 GMAT run | Confirms physics; validates Python numbers |
| 3. Simulate finite | GMAT FiniteBurn + DifferentialCorrector | Scenario 08 | Adds thruster realism; finds burn duration |
| 4. Upload command | Ground operations (TC upload) | NigComSat real ops | Burn duration in seconds to satellite computer |

9. GMAT Lessons Learned

9.1  ImpulsiveBurn vs. FiniteBurn — Critical Differences

| Aspect | ImpulsiveBurn | FiniteBurn |
| --- | --- | --- |
| Hardware required | None — remove hardware braces {} | ChemicalTank + ChemicalThruster |
| Coordinate frame | Local / Earth / VNB (inline syntax) | Must also specify Axes in FiniteBurn |
| Delta-V input | Element1 = Delta-V in km/s directly | Burn duration + thrust + Isp |
| DC needed | No — vis-viva gives exact answer | Yes — no closed-form solution |
| Residual ECC | Near-zero (idealisation) | Non-zero (burn arc effect) |
| Fuel depletion | Not modelled | DecFuelMass = true required |

9.2  VNB Frame Syntax

Correct (R2022a): Create ImpulsiveBurn Burn1; then property lines GMAT Burn1.CoordinateSystem = Local; | GMAT Burn1.Origin = Earth; | GMAT Burn1.Axes = VNB;

Incorrect: Create CoordinateSystem VNB; — VNB is not a named CS; this causes a 'Nonexistent CoordinateSystem' error.

9.3  Stopping Conditions for Manoeuvre Timing

{LEOSat.Periapsis} and {LEOSat.Apoapsis} are the correct GMAT stopping conditions for Hohmann transfer timing. They propagate the spacecraft to the exact orbital geometry rather than using a time-based approximation. This is superior to a fixed elapsed time because J2 perturbation shifts the periapsis passage slightly from the analytically computed time — using the geometry-based condition ensures the burn fires at the exact right orbital position regardless of perturbation-induced timing drift.

9.4  ReportFile — Milestone Output Pattern

The ManoeuvreReport uses explicit Report commands (not the ReportFile Add={} continuous output pattern) to capture only the six milestone states. This produces a clean, readable output file with one data row per milestone preceded by a header row. Each header-data pair is self-contained, making the file easy to parse programmatically or read manually.

Note: GMAT includes a header line before each Report command output — this is the milestone-based report pattern. The continuous ReportFile Add={} pattern would produce thousands of rows (one per integration step) which would obscure the key state changes at burn events.

9.5  GMAT R2022a: Strict ASCII Requirement

Any non-ASCII byte in a GMAT script file causes an Interpreter Exception before parsing begins. This includes: em-dash (from Word/rich-text), degree symbol (deg), Greek letters (Delta, omega), mathematical operators (x for multiplication, approx), and any Unicode character above 0x7F. All script comments must use plain ASCII equivalents: -- for em-dash, deg for degree symbol, Delta for the Greek letter, etc.

9.6  GMAT RAAN Prefix Requirement

In GMAT R2022a, every property line in the Resource section requires the GMAT keyword prefix (e.g., GMAT LEOSat.SMA = 6758.14;). Omitting the prefix causes an interpreter error. This is a non-trivial difference from some other simulation tools and from informal script examples found online that pre-date R2022a.

10. Operational Significance

10.1  LEO Satellite Operations Context

Orbit restoration after drag decay is a routine LEO operations task. The specific scenario modelled here — 20 km SMA correction after a missed station-keeping cycle or anomaly period — directly reflects the responsibilities of an LEO flight dynamics engineer. The key operational skills demonstrated are:

Analytical manoeuvre planning: Deriving the correct Delta-V from first principles without relying on trial-and-error DC iteration. This is the expected approach in an FD team environment.

GMAT scripting proficiency: Configuring impulsive burns correctly in R2022a, including the VNB frame syntax and periapsis/apoapsis stopping conditions.

Result verification: Cross-checking GMAT output against the analytical prediction at each milestone, and interpreting physical deviations (J2 effects) correctly vs. scripting errors.

Error diagnosis: Identifying and correcting GMAT interpreter errors systematically, distinguishing warnings from fatal errors.

10.2  Delta-V Budget Context

11.338 m/s for a 20 km SMA correction is an extremely fuel-efficient operation. For comparison:

| Operation | Typical Delta-V |
| --- | --- |
| Scenario 06: 20 km LEO orbit restoration (Hohmann) | 11.338 m/s |
| Scenario 08: 200 km Hohmann transfer (400->600 km), finite burn + DC | ~108 m/s (10x larger) |
| GEO EWSK (one cycle, ~15 days) | ~0.165 m/s (Scenario 09b: 11s burn) |
| GEO NSSK (one year, lunisolar compensation) | ~50 m/s/year (Scenario 10, planned) |
| LEO->GEO transfer (Hohmann) | ~3900 m/s (for reference) |

The low Delta-V cost of LEO altitude restoration underscores the efficiency of the Hohmann transfer for small corrections. In an LEO constellation, regular 10-20 km SMA corrections can be performed with a tiny fraction of the propellant budget, making them feasible on a monthly or even weekly basis if drag decay warrants it.

10.3  NigComSat-1R Parallel

At GEO, the same two-burn Hohmann principle is used for East-West station-keeping: a prograde burn at perigee raises the apogee (increasing SMA above synchronous, making the period longer than one sidereal day), producing a westward drift arc. A second burn circularises at the new SMA. Scenario 06 (LEO Hohmann restoration) and Scenario 09b (GEO EWSK two-burn cycle) are different operational contexts of the same underlying orbital mechanics — the physics-first methodology applies identically to both.

11. Portfolio Scenario Summary

Scenario 06 forms part of a systematic progression from fundamental LEO propagation through increasingly complex mission operations:

| Scenario | Title | Key Concept | Status |
| --- | --- | --- | --- |
| 01 | LEO Orbit Propagation | Ground track, period, J2 drift | Complete |
| 02 | Multi-Satellite ISL Geometry | 3D constellation setup | Complete |
| 03 | Ground Station Access Windows | ContactLocator, pass prediction | Complete |
| 04a | Two Co-Planar ISL Monitoring | J2 differential TA drift (2 sats) | Complete |
| 04b | Six Co-Planar ISL Analysis | ISL threshold violation, 3 pair classes | Complete |
| 05 | Interplanar ISL Geometry | Cross-plane ISL, 46.5-min TCA period | Complete |
| 06 | Hohmann Orbit Restoration | Two-impulse manoeuvre, vis-viva DV derivation | Complete (this scenario) |
| 07 | Walker 24/3/1 Constellation | 24-sat fleet, ContactLocator at 4 stations | Complete |
| 08 | Finite Burn + Fuel Budget | 400->600 km Hohmann, DC, fuel depletion | Complete |
| 09 | GEO EWSK (Single Burn) | Tesseral harmonics, 42.5 deg E slot | Complete |
| 09b | GEO EWSK (Full Cycle) | Two-burn EWSK cycle, SMA-matched Burn 2 | Complete |
| 10 | GEO NSSK (Lunisolar INC) | Out-of-plane normal burn, 0.75 deg/yr INC growth | Planned |
| 11 | Conjunction Analysis | TCA, miss distance, Pc threshold | Planned |
| 12 | Contact Scheduling + ILMOP | GMAT vs SGP4, ILP scheduler validation | Planned |

Appendix: File Reference

| File | Location | Purpose |
| --- | --- | --- |
| 06_hohmann_transfer_orbit_correction.script | scenario_06_hohmann_orbit_restoration\ | GMAT simulation script (R2022a) |
| Hohmann_Transfer_Delta-V_Calculator.py | scenario_06_hohmann_orbit_restoration\ | Python vis-viva pre-computation script |
| DeltaV_Derivation_Scenario06.docx | scenario_06_hohmann_orbit_restoration\ | Full analytical derivation with equations |
| output\ManoeuvreReport.txt | GMAT_portfolio\ | GMAT milestone report (6 events x 5 columns) |
| Scenario_06_HohmannOrbitRestoration_Analysis.docx | scenario_06_hohmann_orbit_restoration\ | This report document |

End of Scenario 06 Analysis Report

Stephen Ogodo | GMAT Astrodynamics Portfolio | October 2026