GMAT Astrodynamics Portfolio

Scenario 03 — LEO Orbit Propagation, Ground Track &

Ground Station Access Analysis

Author: Stephen O. Ogodo Tool: GMAT R2022a ¦ Date: January 2024 Classification: Portfolio / Educational

# 1. Executive Summary

This report documents Scenario 03 of the GMAT Astrodynamics Portfolio: a 24-hour propagation of a single Low Earth Orbit (LEO) satellite with full ground track visualisation and ground station contact analysis at Abuja, Nigeria. The scenario establishes the core skills required for LEO pass planning — orbital mechanics, ground track geometry, elevation masking, and ContactLocator report generation — that underpin all subsequent multi-satellite scenarios in the portfolio.

The satellite orbits at ~400 km altitude in an ISS-class 51.6° inclined circular orbit. Over 24 hours it completes approximately 15.5 orbits, tracing a sinusoidal lattice across the ±51.6° latitude band with a ~23° westward longitude shift per orbit. GMAT produced three confirmed contact windows with the Abuja ground station above a 5° elevation mask, with a longest pass of 491.7 seconds (8.2 minutes) and total daily contact time of 1,189 seconds (19.8 minutes).

# 2. Mission Objectives

This scenario has three primary objectives:

- Propagate a LEO satellite for a full 24-hour period using a realistic force model (JGM2 4×4 gravity, no drag) and demonstrate correct orbital mechanics through GMAT visualisations.

- Generate and analyse the ground track to confirm the latitude coverage band imposed by orbital inclination and the characteristic westward longitude drift per orbit.

- Compute precise contact windows between the satellite and the Abuja ground station using GMAT's ContactLocator, and interpret the results in the context of NigComSat ground segment operations.

Secondary objectives:

- Demonstrate professional GMAT script authoring practice — fully commented, documented orbital design rationale, and structured for portfolio review.

- Establish the baseline single-satellite contact analysis methodology that is extended to multi-satellite Walker constellations in Scenario 07.

# 3. Orbital Design Parameters

The satellite parameters are drawn from the ISS operational orbit — the most widely recognised LEO benchmark — making the results immediately interpretable by anyone familiar with LEO communications and remote sensing.

| Parameter | Value | Rationale |
| --- | --- | --- |
| Semi-major Axis (SMA) | 6778.14 km | ~400 km altitude — ISS/broadband LEO class |
| Eccentricity (ECC) | 0.0001 (~0) | Near-circular; near-constant altitude throughout orbit |
| Inclination (INC) | 51.6° | Matches ISS; chosen to represent broadband LEO operations |
| RAAN | 0° | Reference ascending node; no preferred longitude required |
| AOP | 0° | Circular orbit — AOP is undefined/arbitrary |
| True Anomaly (TA) | 0° | Satellite starts at equatorial ascending node crossing |
| Epoch | 01 Jan 2024 00:00:00 UTC | Clean reference epoch; aligns with portfolio convention |
| Simulation Duration | 1 day (86400 s) | Full daily coverage cycle; captures all daily passes |

## 3.1 Force Model

The propagator uses the JGM2 gravitational field at degree and order 4×4. This captures the dominant J2 zonal harmonic — responsible for nodal regression (~7°/day westward at 400 km) and apsidal precession — plus the C22/S22 tesseral terms that introduce a slight longitude-dependent acceleration. Atmospheric drag and solar radiation pressure are switched off: the goal is ground track geometry and contact analysis over one day, not altitude decay modelling. The orbit remains at nominally constant SMA (~6778 km) throughout the simulation.

## 3.2 Propagator Settings

Integrator: Runge-Kutta 8(9) adaptive step (RK89)

Initial step size: 60 s

Accuracy tolerance: 1×10⁻¹² (relative error per step)

Min / Max step: 0.001 s / 2700 s

Error control: RSSStep (root-sum-square)

# 4. Ground Station Configuration

Abuja, the Federal Capital Territory of Nigeria, is the primary ground station for this scenario. It represents the NigComSat ground segment — a directly relevant operational context given the author's 15-year career at Nigerian Communication Satellite Ltd (NigComSat).

| Parameter | Value |
| --- | --- |
| Location | Abuja, Federal Capital Territory, Nigeria |
| Geodetic Latitude | 9.0579° N |
| East Longitude | 7.4951° E |
| Altitude | 342 m above WGS-84 ellipsoid |
| Min Elevation Angle | 5° (operational mask for LEO tracking) |
| Horizon Reference | WGS-84 Ellipsoid |
| Light-time Correction | Enabled (Transmit direction) |
| Stellar Aberration | Disabled |

Abuja sits at 9.06°N — well within the ±51.6° latitude coverage band imposed by orbital inclination. Passes do occur, but the ~23°/orbit westward ground track drift means only a subset of the ~15.5 daily orbits pass close enough to Abuja's horizon to produce an access event above the 5° mask.

# 5. GMAT Script Architecture

The script (`03_LEO_Orbit_Propagation_commented.script`) is organised into six sections:

| Section | Contents |
| --- | --- |
| Spacecraft | LEOSat — Keplerian ICs, physical properties, NAIF IDs, attitude model |
| GroundStations | Abuja — geodetic coords, elevation mask, atmosphere model (off) |
| ForceModels | InternalODEModel — JGM2 4×4 gravity, no drag, no SRP, no relativity |
| Propagators | DefaultProp — RK89, 60 s initial step, 1×10⁻¹² accuracy |
| EventLocators | AbujaAccess — ContactLocator, 10 s search step, UseEntireInterval=true |
| Subscribers | LEOView (OpenFramesInterface 3-D orbit) + GroundTrack (2-D Mercator) |

The mission sequence is a single command: `Propagate DefaultProp(LEOSat) {LEOSat.ElapsedDays = 1}`. GMAT runs the ContactLocator automatically during propagation (RunMode = Automatic), writing the access report to disk at the end of the run. The UseEntireInterval = true setting ensures the full 24-hour window is searched regardless of propagation stepping.

# 6. Orbital Mechanics Analysis

## 6.1 Derived Orbital Quantities

| Quantity | Value | Formula / Source |
| --- | --- | --- |
| Orbital Period (T) | 5553 s (92.55 min) | T = 2π√(a³/μ), μ = 398600.4 km³/s² |
| Mean Motion (n) | 0.001131 rad/s | n = 2π/T |
| Orbital Velocity (v) | 7.669 km/s | v = √(μ/a) |
| Orbits per Day | ~15.56 | 86400 s / 5553 s |
| Altitude | ~400 km | SMA − R⊕ (R⊕ = 6378.14 km) |
| Latitude Coverage Band | ±51.6° | Hard ceiling = inclination angle |
| Earth Rotation Rate | 0.25°/min (360°/day) | Sidereal rate ≈ 0.2507°/min |
| Westward Track Shift per Orbit | ~23.1° | 0.25 °/min × 92.55 min |

## 6.2 Orbital Period Derivation

The orbital period follows directly from Kepler's third law:

T = 2π × √(a³ / μ)

T = 2π × √(6778.14³ / 398600.4)

T = 2π × √(311,001,500 / 398600.4)

T = 2π × √(780.20)  =  2π × 27.93  =  175.49 rad × (1 s/rad)  ≈  5553 s  =  92.55 min

## 6.3 Ground Track Geometry

The sinusoidal ground track pattern emerges from the combination of orbital motion (inclination 51.6°) and Earth's rotation beneath the orbit plane. Each successive ascending node crossing occurs ~23.1° west of the previous one because Earth rotates ~23.1° eastward during one 92.55-minute orbit. After ~15.5 orbits the pattern nearly repeats (0.5 orbit residual means the satellite is on the opposite side of Earth at 24 h compared to the start).

The latitude coverage band is a hard ceiling: the sub-satellite latitude oscillates between −51.6° and +51.6° exactly, never exceeding the orbital inclination. This is why polar regions (above ~52°) are inaccessible to this orbit — a fundamental constraint that drives the inclination choices for Tromso-covering constellations.

## 6.4 Nodal Regression (J2 Effect)

The dominant non-Keplerian perturbation at LEO is J2 — the equatorial bulge of Earth. It causes the orbital plane (RAAN) to precess westward at a rate dependent on inclination:

dΩ/dt = −(3/2) × n × J2 × (R⊕/a)² × cos(i) / (1 − e²)²

dΩ/dt ≈ −6.96°/day  for i = 51.6°, a = 6778.14 km

Over the 24-hour simulation this introduces a ~6.96° westward shift in the ascending node — observable as a very slight eastward drift in the apparent repeat pattern of the ground track. The 4×4 JGM2 force model captures this accurately.

# 7. Contact Analysis Results

GMAT's ContactLocator computed all access events during the 24-hour window (01 Jan 2024 00:00:00 UTC to 02 Jan 2024 00:00:00 UTC) where the satellite elevation above Abuja exceeded the 5° mask. The algorithm uses root-finding interpolation rather than discrete sampling, so durations are accurate to millisecond precision.

| Pass | Start (UTC) | End (UTC) | Duration (s) | Duration (min) |
| --- | --- | --- | --- | --- |
| 1 | 01 Jan 2024 03:43:00.095 | 01 Jan 2024 03:50:28.597 | 448.50 | 7.48 |
| 2 | 01 Jan 2024 05:20:39.019 | 01 Jan 2024 05:24:47.990 | 248.97 | 4.15 |
| 3 | 01 Jan 2024 16:54:43.360 | 01 Jan 2024 17:02:55.083 | 491.72 | 8.20 |
| TOTAL |  |  | 1189.19 | 19.82 |

## 7.1 Contact Summary Statistics

| Metric | Value |
| --- | --- |
| Total passes (24 h) | 3 |
| Longest pass | 491.72 s (8.20 min) — Pass 3 |
| Shortest pass | 248.97 s (4.15 min) — Pass 2 |
| Mean pass duration | 396.4 s (6.61 min) |
| Total daily contact time | 1189.2 s (19.82 min) |
| Longest gap (Pass 2 → Pass 3) | ~11 h 34 min |
| Daily contact fraction | ~1.38% of 24-hour period |
| Elevation mask applied | 5° minimum |

## 7.2 Physical Interpretation

Three passes in 24 hours is consistent with the orbital geometry. Of the ~15.5 daily orbits, the ~23°/orbit westward ground track shift means most ascending nodes cross the equator far from Abuja's longitude (~7.5°E). Only three orbits pass close enough to bring the satellite above the 5° elevation horizon.

Pass 2 (248.97 s) is noticeably shorter than Passes 1 and 3, indicating a low-elevation geometry — the satellite skims near the edge of the visibility cone rather than passing nearly overhead. Pass 3 (491.72 s, 8.2 min) represents a near-overhead pass with maximum elevation well above the 5° mask.

The 11.6-hour gap between Passes 2 and 3 highlights a critical operational consideration: a single LEO satellite cannot provide continuous coverage to any single ground station. This coverage gap drives the constellation design philosophy explored in Scenario 07 (Walker 24/3/1), where 24 satellites reduce the maximum revisit gap to under 12 minutes.

# 8. Operational Significance

This scenario directly mirrors the type of ground station access analysis performed during the author's tenure at NigComSat, where pass windows for the NigComSat-1R GEO satellite were tracked from the Abuja ground segment. The LEO context differs fundamentally from GEO operations:

| Aspect | GEO (NigComSat-1R) | LEO (This Scenario) |
| --- | --- | --- |
| Visibility | Continuous (always above horizon) | Intermittent — 3 passes / day |
| Pass duration | Permanent (24 h/day) | 4–8 minutes per pass |
| Contact fraction | ~100% | ~1.4% of day |
| Altitude | 35,786 km | ~400 km |
| Latency (one-way) | ~119 ms | ~1.3 ms |
| Station-keeping | Required (EWSK/NSSK) | Natural orbit decay only |

The low contact fraction (~1.4%) for a single LEO satellite over Abuja underscores why the LEO broadband industry deploys large constellations (Starlink: ~6,000 satellites; OneWeb: 648 satellites) to achieve near-continuous global coverage. The same physics that gives LEO its latency advantage — proximity to Earth — creates the coverage gap problem that only scale can solve.

# 9. Key Equations & Derivations

## 9.1 Orbital Period (Kepler's Third Law)

T = 2π √(a³/μ)

where a = semi-major axis (km), μ = 398600.4418 km³/s² (Earth gravitational parameter).

## 9.2 Orbital Velocity (Circular Orbit)

v = √(μ/a)  =  √(398600.4 / 6778.14)  =  7.669 km/s

## 9.3 Westward Ground Track Shift per Orbit

ΔΛ = ωE × T

ΔΛ = (360°/86164 s) × 5553 s  =  23.18°/orbit

where ωE = Earth's sidereal rotation rate = 360°/86164 s = 0.004178°/s.

## 9.4 J2 Nodal Regression Rate

dΩ/dt = −(3/2) n J2 (R⊕/a)² cos(i) / (1 − e²)²

J2 = 1.08263 × 10⁻³,   R⊕ = 6378.14 km,   i = 51.6°

dΩ/dt  ≈  −6.96°/day

## 9.5 Maximum Theoretical Pass Duration

For a satellite at altitude h = 400 km passing directly overhead (max elevation = 90°), the maximum geometrically possible pass duration is:

2 × (1/n) × arccos[ R⊕ cos(ε_min) / (R⊕ + h) ]

where ε_min = 5°  (elevation mask),  n = mean motion = 0.001131 rad/s

≈  2 × (1/0.001131) × arccos(6378.14 × cos5° / 6778.14)

≈  2 × 884 × arccos(0.9398)  ≈  2 × 884 × 0.3539  ≈  626 s  (10.4 min)

The longest observed pass of 491.7 s (7.8 min) is below this theoretical maximum, confirming the satellite did not pass directly overhead — it was off to the side of Abuja's zenith.

# 10. Conclusions

Scenario 03 successfully demonstrates a complete LEO single-satellite ground station access analysis in GMAT R2022a. The key findings are:

- A single 51.6°-inclined LEO satellite at 400 km altitude provides only 3 contact windows per day to Abuja (9°N), with total daily contact of ~19.8 minutes — approximately 1.4% of the day.

- Pass durations range from 4.15 to 8.20 minutes, consistent with the theoretical maximum of ~10.4 minutes for a 5° elevation mask at 400 km altitude.

- An 11.6-hour gap between consecutive passes (Passes 2 and 3) represents a critical operational discontinuity that single-satellite LEO systems cannot resolve.

- The ~23°/orbit westward ground track shift, driven by Earth's rotation, is the geometric cause of the sparse pass schedule — most of the 15.5 daily orbits cross the equator far from Abuja's longitude.

- The JGM2 4×4 force model correctly reproduces J2 nodal regression (~7°/day), giving a physically accurate ground track evolution over 24 hours.

These results establish the single-satellite baseline against which Scenario 07's Walker 24/3/1 constellation — achieving 128 daily passes to London and near-continuous coverage — should be benchmarked. The physics demonstrated here (inclination ceiling, westward drift, contact fraction) are invariant across all subsequent LEO scenarios in this portfolio.

End of Report — Scenario 03

GMAT Astrodynamics Portfolio  |  Stephen O. Ogodo  |  2024